11.2 Non-Conventional Sources of Energy 705

11.2 NON-CONVENTIONAL SOURCES OF ENERGY

Introduction

The Industrial Revolution of the 19

th

century ushered in new

technologies.  The spurt in inventions in that century was unprecedented in

many ways.  Some of these inventions involved use of natural resources like

coal and oil.   The thought of exhaustible nature of these resources and the

environmental damage from the use of these resources never occurred either

to the inventors or the subsequent generations.  In the quest to sustain

galloping economic activity, the dependence on coal and oil has soared at a

phenomenal rate over the years.  The burnt fuels result in the release of

carbondioxide and other gases into the atmosphere causing environmental

damage. It has become imperative to look at energy technology with a new

perspective.  There are abundant renewable sources of energy such as wind,

sun, water, sea, biomass apart from even daily wastes.  These sources are

pollution free and hence clean energy apart from being unlimited/

inexhaustible.

Power generation in India has grown in size to around 1 lakh MW and

in Tamil Nadu it has increased to 7924 MW which is distributed through a vast

network of transmission, sub-transmission and distribution lines that reach all

villages  even in remote areas.  The demand for power is growing rapidly.

The problem will be compounded due to fast depletion of fossil fuel deposits,

quality of fuels, heavy price to be paid for basic materials plus their

transportation cost and above all the environmental degradation caused by

the use of conventional energy sources.  Under such conditions, environmentfriendly and pollution-free, non-conventional and renewable energy sources

known as 'clean and green energy' have emerged as an important

alternatives to conventional energy sources. The renewable energy sources

are clean and inexhaustible as they rely on sun, wind, biomass, etc., as

primary sources of energy.  It is estimated that, about 2000 MW can be

generated from wind potential available in Tamil Nadu.  As against this

potential, 19 MW of power in the State Sector mostly through demonstration

wind farms and 838 MW in the private sector have been harnessed as on

31.3.02,   Under Biomass, the estimated potential is about  500 MW and 154

MW capacity has been expected using biomass/bagasse.

The country is endowed with large amount of sustainable resource

base and non-conventional energy technologies which are well-suited for grid

connected power generation,  energy supplies in remote areas which are  not/

could not be connected to the grid and for captive consumption.  Nonconventional energy sources like wind energy, solar energy through thermal

as well as photovoltaic system, biomass and hybrid sources will help to a

great extent in enhancing power generation capacity.   Hence   appropriate

policies and programmes that optimise the use of available energy resources

with new technologies have to be propagated, promoted and adopted, if

necessary, by budgetary support.

Tamil Nadu has done very well in promoting and harnessing renewable

sources of energy particularly wind and  bagasse based power generation.

With a view to develop and propagate the non-conventional sources of706 11.2 Non-Conventional Sources of Energy

energy,  the Tamilnadu Energy Development Agency (TEDA)  was formed in

1985. Apart from serving as a coordinating agency to promote the use of non

conventional energy sources, TEDA acts as nodal agency to the Ministry of

Non-conventional Energy Sources (MNES), Government of India to

implement Centrally funded and sponsored schemes in the State.  During the

past 17 years, TEDA has conducted sustained campaigns to promote

harnessing of renewable energy with active support from the Central and the

State Governments.  The total installed capacity in Tamilnadu under Nonconventional Energy Sources is 1000 MW as on 31.3.2002 which is about

13% of total TNEB grid capacity, while at the  All-India level, it is only 3%.

The major share of this comes from Wind Energy followed by bagasse based

co-generation  plants  in sugar industries.  This has largely come  through

private investments encouraged by policy initiatives of the Central and the

State Governments.

Non - Conventional or Renewable Energy Sources

1.  Wind Energy

The evolution of windmills into wind turbines did not happen overnight

and  attempts to produce electricity with windmills date back to the beginning

of the century.  It was Denmark which erected the first batch of steel windmills

specially built for generation of electricity. After World War II, the development

of wind turbines was totally hampered due to the installation of massive

conventional power stations using fossil fuels available at low cost.  But the oil

crisis of 1973 heralded a definite break through in harnessing wind energy.

Many European countries started pursuing the development of wind turbine

technology seriously and their development efforts are continuing even today.

The technology involves generation of electricity using turbines, which

converts mechanical energy created by the rotation of blades into electrical

energy, some times the mechanical energy from the mills is directly used for

pumping water from well also.  The wind power programme in India was

started during 1983-84 with the efforts of the Ministry of Non-Conventional

Energy Sources.  In India the total installed capacity from wind mills is 1612

MW, of which, Tamilnadu has an installed capacity of 858 MW as on

31.03.2002.

Tamil Nadu is endowed with lengthy mountain ranges  on its Western

side with three prominent passes in its length.  These are with wind-potentials:

(1) Palghat Pass in Coimbatore District-1200 MW, (2) Shengottah Pass in

Tirunelveli District-500MW and (3) Aralvoymozhi Pass in Kanniyakumari

District- 300 MW (Total potential-2000 MW).  The mountainous areas  close to

Cumbum Valley are observed to be having high potential and, though coastal

areas, central plains and hilly areas have been observed unsuitable for wind

power projects, Rameshwaram  is found suitable.

2.  Bio Energy

Biomass is yet another important source of energy with potential to

generate power to the extent of more than 50% of the country’s requirements.

India is predominantly an agricultural economy, with huge quantity of biomass

available in the form of husk, straw, shells of coconuts wild bushes etc.  With 11.2 Non-Conventional Sources of Energy 707

an estimated production of 350 million tons of agricultural waste every year,

biomass is capable of supplementing coal to the tune of about 200 million

tonnes producing 17,000 MW of power and resulting in a saving of about

Rs.20,000 crores every year.  Biomass available in India comprises of rice

husk, rice straw, bagasse, coconut shell, jute, cotton, husk etc.    Biomass can

be obtained by raising energy farms or may be obtained from organic waste.

The biomass resources including large quantities of cattle dung can be used

in bio-energy technologies viz., biogas, gasifier, biomass combustion, cogeneration etc., to produce energy-thermal or electricity.   Biomass can be

used in three ways  – one in the form of gas through gasifiers for thermal

applications,  second in the form of methane gas to run gas engines and

produce power and the third through combustion to produce steam and

thereby power.

3.  Solar Energy

Solar Power was once considered, like nuclear power, ‘too cheap to

meter’ but this proved illusory because of the high cost of  photovoltaic cells

and due to limited demand.  Experts however believe that with mass

production and improvement in technology, the unit price would drop and this

would make it attractive for the consumers in relation to thermal or hydel

power.  The Solar Photo Voltaic (SPV) technology which enables the direct

conversion of sun light into electricity can be used to run pumps, lights,

refrigerators, TV sets, etc., and it has several distinct advantages, since it

does not have moving parts, produces no noise or pollution, requires very little

maintenance  and can be installed anywhere.  These advantages make them

an ideal power source for use especially in remote and isolated areas which

are not  served by conventional electricity making use of ample sunshine

available in India,  for nearly 300 days in a year.

A Solar Thermal Device, on the other hand captures and transfers the

heat energy available in solar radiation.  The energy generated can be used

for thermal applications in different temperature ranges.   The heat can be

used directly or further converted into mechanical or electrical energy.

4. Other Sources

The other sources of renewable energy are geothermal, ocean,

hydrogen and fuel cells.  These have immense energy potential, though

tapping this potential for power generation and other applications calls for

development of suitable technologies.

(i)  Geo-Thermal Energy

Geo-Thermal energy is a renewable heat energy from underneath the

earth.  Heat is brought to near surface by thermal conduction and by intrusion

into the earth’s crust.  It can be utilised for power generation and direct heat

applications.  Potential sites for geo-thermal power generation have been

identified mainly in central and northern regions of the country.   Suitable

technologies are under development to make its exploitation viable.708 11.2 Non-Conventional Sources of Energy

(ii) Ocean thermal and Tidal energy

The vast potential of energy of the seas and oceans which cover about

three fourth of our planet, can  make a significant contribution to meet the

energy needs.  Ocean contains energy in the form of temperature   gradients,

waves and tides and ocean  current, which can be used to generate electricity

in an environment-friendly manner.   Technologies to harness  tidal power,

wave power and ocean thermal energy are being developed, to make it

commercially viable.

(iii) Hydrogen and Fuel Cells

In both Hydrogen and Fuel Cells  electricity is produced through an

electro-chemical reaction between hydrogen and oxygen gases.  The fuel

cells are efficient, compact and reliable for automotive applications.  Hydrogen

gas is the primary fuel for fuel cells also. Hydrogen can be produced  from the

electrolysis of water using solar energy.  It can also be extracted from sewage

gas, natural gas, naptha or biogas.  Fuel cells can be very widely used once

they become commercially viable.

(iv)    Bio fuels

In view of worldwide demand for energy and concern for environmental

safety there is need to search for alternatives to petrol and diesel for use in

automobiles.   The Government  of India has now permitted the use of 5%

ethanol blended petrol.  Tamilnadu is one of the nine States in the country

where this programme will commence from January 2003.  Ethanol produced

from molasses/ cane juice, when used as fuel will reduce the  dependence on

crude oil and help  contain pollution.   Further, technology is also being

developed to convert different vegetable oils especially non-edible oils  as

bio-diesel for use in the transport sector.  They are however, in R & D stage

only.

5.  Potential  and Exploitation of Renewable Energy Sources

India ranks fifth in the world in Wind power with installed capacity of

1612 MW out of an estimated potential of 45,000 MW.  Tamilnadu  ranks first

in the country in Wind power with a capacity of 858 MW out of an estimated

potential of 3050 MW.   In biomass power the country has an installed

capacity of 381 MW out of  total potential of 19500 MW.  In Tamilnadu the

installed capacity is 142 MW against the  potential of 1000 MW. The potential

available under solar photovoltaic energy is  20 MW per Sq.Km.  But in view

of high cost and heavy investment involved the progress is rather slow.

In Solar thermal energy (Solar Water Heater system) 15 lakh M

2

collector area has been installed in the country against a potential of 1400

lakh M

2

In Tamilnadu, 20084 M   .

2

area has been installed.  There is

considerable scope for expanding this activity with suitable incentives.

The most note-worthy achievement of Tamilnadu has been in creating

an installed capacity of  about 1000 MW from the non-conventional energy

sources alone in the State, i.e., 13% of the total TNEB  grid capacity against

3.2% only for the country.  The major component of this has come from Wind

Energy (858 MW) followed by co-generation in sugar industries (142 MW).

Further, this has largely come about through private investment due to11.2 Non-Conventional Sources of Energy 709

attractive policy initiatives of the State and Central Governments.  It  may be

worthwhile to offer various incentives to enhance its share further in view of

the vast potential available.

Ninth  Five Year Plan Performance

An outlay of Rs.19.95 crores was approved for  Non- Conventional

Energy sector for the Ninth Plan.  But, the actual allocation by the State

Government was Rs.8.73 crores which is 44% of the outlay, of which Rs.8.57

crores was spent.  The over all financial and physical performance   is shown

below:

Financial Performance

Rs. in lakhs                                     Rs. in lakhs

Sl.

No.

Scheme Ninth

Plan

outlay

1997-

2002

Ninth Plan

actual funds

allocated by

State Govt.

% of funds

allotted to

total

outlay

Total 9

th

Plan

perform

ance

% Performance to

total funds

allotted

1. Wind Energy  374.40  15.22 4.1%  14.42 94.7%

2. Solar Energy  283.60 210.42 74.2% 208.92 99.3%

3. Bio Energy  531.40  43.45 8.2%  36.49 84.0%

4. IREP  459.00 300.87 65.5% 298.42 96.0%

5. Others  346.70 302.88 87.4% 298.87 98.5%

Total 1995.10 872.84 43.8% 857.12 98.2%

Physical Performance

Sl.

No.

Scheme Unit 9

th

Five Year Plan

Target  Achievement

1. Windmill for Water Pump Nos. 22 18

2. Wind Resource Monitoring Station Nos. 11 11

3. Wind Energy Project

Public

Private

MW

-

-

-

181

4. Biogas plant Nos. 14 6

5. Gasifier Nos. 13 16

6. Taluka Level Biomass Assessment Study Nos. 4 4

7. Assessment Study on Municipal Waste Nos. 5 5

8. Biomass Based Power Project MW - -

9 Co-generation in sugar mill MW - 69(private)

10. Solar Thermal

Domestic

Industrial

Government Institutions

Nos.

Sq.m

Nos.

1250

1240

12

1370

1280

12

11. Solar Still Sq.M

.

400 -

12. Solar Photovoltaic

Street Light

Pump

Lantern

Roof Top Power Plant

Nos.

Nos.

Nos.

KW

60

95

1000

11

60

95

1000

9*

*Includes 4 KW installed in TEDA office at Chennai.710 11.2 Non-Conventional Sources of Energy

1.  Wind Energy

The total wind potential in the State is around 2000 MW of which about

40% only has been tapped so far. The installation of 119 Wind electric

generators for a total capacity of 19 MW as part of demonstration

programmes has motivated and encouraged private sector to install 3003

Wind electric generators for a total capacity of  838 MW.   The Wind Mills

have a total capacity of 857 MW, of which, 181 MW capacity was added

during the Ninth Plan and have generated and fed into the grid (6816.379 Mu.)

Installation of windmill for water pumping was taken up by providing

subsidy ranging from Rs.20,000 to  Rs.32,000 from State Government

besides MNES subsidy of Rs.30,000 to Rs.45,000 against the total cost of

Rs.80,000 for gear type and Rs.1,45,000 for AV55 type Wind Mills.  During

Ninth plan period, 18 gear type Wind Mills were installed from 1997 to 2000

and another 4 mills were also installed by the end of 2002.   Wind monitoring

stations were set up in 11 centres in Coimbatore, Erode, Kanyakumari and

Tuticorin to conduct Wind Resource Assessment study which helps to identify

potential locations for wind mills.

2.   Bio-Energy

It is technically possible and economically feasible to generate biogas

from cattle dung and certain agro and industrial waste.   During Ninth Plan, 12

Biogas plants were  installed in livestock farms, educational institutions,

agricultural farms, central prisons and sugar mills.   24 Gasifier systems  i.e.,

23 thermal gasifiers and one  electrical gasifier  were also installed  during

Ninth Plan with financial assistance from the Ministry of Non-conventional

Energy  Sources.

Resource Assessment studies were carried out in 35 Taluks under

MNES Programme and  4 Taluks under State programme during Ninth Plan.

This will be useful for private entrepreneurs to evolve suitable power projects.

The sugar mills go in for co-generation projects by upgrading their

boilers and other accessories suitably so as to supply excess electricity to

Tamilnadu Electricity Board.   Government of India extends  financial

assistance up to Rs.45 lakh per MW of surplus power to Co-operative sugar

mills and interest subsidy to the Private Sugarmills.  At present, the installed

capacity through co-generation in sugarmills is 142 MW.  Other potential

sectors for co-generation are cement, paper, caustic soda, textiles, iron and

steel etc., with a total potential of 5600 MW in the country.

The Ministry of Non-conventional Energy Sources sanctioned a

scheme for establishing a 1.2 MW Power plant at Sarkar Uduppam Village in

Namakkal District under private sector for energy recovery from poultry

wastes as a demonstration project with an assistance of Rs.3.5 crores which

will be commissioned during the year 2002.  Another demonstration  High

Biomethnation Project has also been sanctioned under MNES for setting up of

a sub-project under UNDP/GEF assisted project for processing of tapioca

wastes water for generation of power (500 Kw) and manure in Pappireddipatti

of Dharmapuri district under private sector.  The project cost is Rs.3.59 crores

which will be shared equally by MNES and the beneficiary and the project will

be commissioned during the year 2002.11.2 Non-Conventional Sources of Energy 711

3.  Solar  Energy

(a) Solar Water Heating System

Several designs and capacities of Solar Water Heating System

(SWHS) are available to suit the users' needs.  The Government of Tamilnadu

provided a subsidy of Rs.2000/- to Rs.3000/- for domestic Water Heating

System from 1993 to 1998.   The Government also sanctioned the full cost of

SWHS in 12 Government buildings, mostly hostels and hospitals, in addition

to which during the Ninth Plan,  1370  domestic and 58  Industrial Water

Heaters were installed in Tamil Nadu.

(b)   Solar Air Heating Systems

Tamilnadu has taken a lead role in the country to promote the use of

Solar Air Heating technology for Tea/ Fruit processing and Grain drying.  14

systems were installed in Tamil Nadu with MNES subsidy.  This process of

pre-heating using solar energy saves nearly 25% of fossil fuels or firewood.

The concept of solar air heating was evolved only in Tamilnadu for the first

time in the country.  The air gets heated when passed through the solar air

heating system, which is used for bringing down the moisture content in the

materials to be dried.  In the process, it provides substantial fuel savings to

the Industries. During  Ninth Plan, 14 solar air heating Systems were installed

in Tamilnadu with the research experience of Planters Energy Network, an

associate of the Madurai Kamaraj University.

(c)  Solar Photovoltaic Systems

Solar Photovoltaic pump is a system which produces  electricity from

sunlight and operates the pump to lift water from  wells.  A 900 watts pump

can deliver about 50,000 litres of water per day over a total head of 30 feet.  It

can irrigate one to two acres.   Under these conditions, the SPV pumps offer a

reliable alternative for small farmers.  The cost of a 900 watts pump is Rs.2.24

lakhs.   IREDA, Government of India  and  GOTN provide a subsidy of

Rs.1,12,500/- at Rs.125/- per watt and Rs.59,000/- per pump respectively and

the remaining is the farmer's contribution.   During  Ninth Plan period, 95

numbers  of  SPV  pumps were installed.

SPV lantern is a portable lighting device which provides light equivalent

to a 60 watt bulb for about 4 hours per day. This is very useful in remote areas

where there is no electricity or where there is frequent failure of electricity.

MNES, Government of India  provides subsidy of Rs.1500 per lantern.  During

the Ninth Plan period, 1000 SPV lanterns were distributed.

SPV Street light is a stand-alone street light system with a 11 W CFL

Lamp.  It provides lighting for 12 hours.    The cost of each light is around

Rs.30,000/-. Tamilnadu Government provided a subsidy of Rs.12000/- per

light.  During the Ninth Plan period 60 SPV  Street Lights were installed.

4. Integrated Rural Energy Planning  (IREP)

The Objectives of the IREP Programme are:

(i) To provide for minimum domestic energy needs for cooking,

heating and lighting purposes to rural people.712 11.2 Non-Conventional Sources of Energy

(ii) To provide the most cost effective mix of various energy sources

and options for meeting the requirements of sustainable agriculture

and rural development with due environmental considerations.

(iii) To ensure people’s participation in planning and implementation.

(iv) To develop and strengthen mechanisms and co-ordination

arrangements for linking micro level planning for rural energy with

national and State level planning and programmes.

The IREP Programme is being implemented by the Tamilnadu Energy

Development Agency in 21 selected blocks all over Tamilnadu.  Under this

programme, the optimum mix of all types of energy locally available is

provided to the rural public for meeting their lighting, cooking and heating

needs.  The programme is funded by Government of India which meets the

staff cost and the State Government bears the scheme cost.   The

programmes  implemented in the Ninth Plan period with subsidies ranging

from 25% to 100% are:

1) Improved Chulah (100% subsidy)

2) Frictionless footvalves (75% subsidy)

3) Solar Cookers (25%) subsidy.

4) Windmills for pumping water for community use (100% subsidy)

5) Other energy saving devices (25% subsidy)

5.  Energy Conservation and Audit

Energy Audit is a systematic approach for effecting energy

conservation in an industry.   Energy audit helps in identifying and assessing

potential areas where energy could be conserved.

TEDA is co-ordinating with TNEB for implementation of Energy Audit in

power intensive industries in Tamil Nadu.

The Government has made Energy Audit mandatory for the industrial

sector and commercial sector with high tension power consumption with

maximum demand exceeding 200 kVA under Phase I and between 1000 kVA

to 2000 kVA under Phase II.  (The new HT services covered under mandatory

Energy Audit programmes are exempted from conducting energy audit for a

period of 3 years from the date of service connection).

TNEB received Energy Audit Reports on 181 HT industries (having a

demand of more than 2000 kVA) covered under Phase I from the registered

Energy Auditors.  Out of these, implementation schedule for 102 Nos. has

been received.  Savings in energy achieved so far is 1494 lakh units / annum.

The total projected saving is 2198 units,  i.e. a capacity equivalent of 25 MW

of Rs.125 crores.

6. Publicity

Tamilnadu Energy Development Agency conducts seminars,

exhibitions, business meets etc. using press and media and also  participates

in the seminars, meets, exhibitions sponsored by various Agencies in order to 11.2 Non-Conventional Sources of Energy 713

propagate the usefulness of the Non conventional Sources of Energy. It is

very essential to create awareness of the benefits derived by the use of NCES

to reach the urban public and also to be taken to the root level in rural areas

by organising energy fairs and exhibitions.  One Mobile Exhibition Van is

available in Tamilnadu Energy Development Agency, which is sent to

exhibitions, Government functions like inaugural functions, festivals and

meetings for publicity purposes throughout State.

7. Evaluation Studies

To evaluate the working of these systems and to bring forth the

bottlenecks and to find out remedy to make the systems work successfully,

periodical evaluation studies were  conducted by TEDA to prove the success

or failure of the programme so as to decide the continuance or discontinuance

of the  programme.  During the Ninth Plan period the following Evaluation

Studies were conducted by TEDA:

(i) Biogas Plants 117 Nos.

(ii) SPV Pumps 100 Nos.

(iii) Wind mill for Water Pumping 108 Nos.

(iv) Wind Electric Generators – 2599 Nos.(711 MW)

(v) SPV Street Lights  – 108 Nos. Solar Water Heating Systems:

Domestic  –100 Nos.; Industrial  – 10 Nos.; and Government

Buildings – 14 Nos.

Bottlenecks and Constraints

While the performance under the renewable energy sources

programmes upto Eighth Five Year Plan was relatively good, the pace of

implementation suffered during the Ninth Plan period due to  various

constraints, some of which are as follows:

(i) Renewable Energy Technologies are capital intensive and require

high initial investment which  investors could not mobilize in the

absence of financial support including capital subsidy from the

Central Government/ State Government.

(ii) The power purchase policy has not been encouraging for private

entrepreneurs and suitable policy initiatives in the form of wheeling,

and banking facilities, evacuation, arrangements,  land allotment

etc.,  need to be considered to boost investment.

(iii) Technologies for several renewable energy sources have not fully

stabilized which has hampered the development and hence  more

intensive R&D efforts are called for with special focus on

partnership with industry.

(iv) Since the cost of renewable energy gadgets/ devices such as solar

cooker, biogas, solar geysers, solar lanterns etc., is high, it is not

possible to generate sufficient demand for these items  though the

people are aware of the advantages.  Hence, the cost needs to be

reduced through suitable support to manufacturers. 714 11.2 Non-Conventional Sources of Energy

(v) Fiscal incentives namely 100% depreciation attracted several

private investors.  But it has been diluted to a considerable extent

by the imposition of Minimum Alternative Tax.

(vi) Adequate number of professionally skilled manpower has not been

developed in the renewable energy sector and hence training

programmes may be organized to develop required manpower.

Tenth Five Year Plan

Vision

ß Provide and promote “clean and green energy” on much wider scale

covering villages and towns to meet the decentralized energy

requirements in agriculture, small scale industries, commercial

establishments and households with priority for remote habitations

which do not enjoy grid power.

ß Enhance the generation of grid quality power through  private

investment for harnessing various renewable energy sources.

ß Encourage energy efficient buildings which will conserve energy to

meet energy requirements from naturally available resources.

ß Promote energy efficiency and energy conservation in industries

domestic use etc.

Goals and Objectives

The national goal of meeting 10% of grid capacity from renewable

sources by 2010 has been achieved by Tamil Nadu (12.5%) even as on

31.3.02.  Under the Tenth Plan, the goal is to consolidate and stabilize the

share of grid connected power with addition of 550 MW and also achieve

decentralized power generation to meet the local energy needs in agriculture,

agro- processing, households  etc., especially in remote areas and expand

the use of renewable energy sources and promote energy efficiency and

thereby energy saving.

Strategies

i. Encourage and promote private investments in renewable energy

through suitable policy initiatives at State level.

ii. Involve local bodies in developing decentralized power and its use in

agriculture, household sectors etc.

iii. Establish field units to promote renewable energy at local levels   by

integrating existing programme staff.

iv. Enable suitable revision of power purchase rate for grid connected

power to make it  attractive for the investors.

v. Encourage research and development  to improve efficiency of the

devices and bring down the cost.

vi. Undertake awareness campaigns in Districts through seminars,

exhibition, etc.11.2 Non-Conventional Sources of Energy 715

New initiatives

The following new initiatives are proposed during the Tenth Plan:

i) Undertake further studies for micrositing for setting up of Wind Mills

by identifying proper and suitable locations

ii) Undertake Biomass potential assessment studies at Taluk level and

make data on Biomass potential available for prospective investors

iii) Merge the staff under Centrally sponsored schemes like IREP and

National  Biogas Development programme and create field outfits to

provide guidance and support to local bodies in tapping renewable

energy.

iv) Popularise and propagate renewable energy use among industries

and households in rural and urban  areas

v) Secure proper and reasonable price for grid connected power

through State Electricity Regulatory Commission

vi) Arrange for suitable adjustments in wheeling and banking facilities,

and third party sale to attract further investment in renewable

energy sector and

vii) Designate TEDA as single window agency to facilitate smooth

clearance for projects upto certain capacity say 25 MW.

Programmes for Tenth Five Year Plan (2002-07)

A.  Wind Energy

i. Wind Power Generation  - It is proposed to add 250 MW wind power

generation through private sector during  Tenth Plan period, if policy

measures, especially power purchase price, are taken in accordance

with  the  guidelines of the Ministry of  Non-conventional Energy

Sources (MNES).

ii. Micro-Level Study- To provide accurate data to the entrepreneurs, it is

proposed to conduct micro-level study  through C-WET in some of the

unexplored regions in the State particularly in hilly terrains.  An outlay

of Rs.10 lakhs is proposed for the Tenth Plan at Rs. 2 lakhs each for 5

locations.

iii. Maintenance of Wind Monitoring Stations  - An amount of Rs.32,000/-

per annum  is proposed for the maintenance of 3 existing permanent

wind monitoring stations in Tamil Nadu by Tamil Nadu Energy

Development Agency (TEDA). An outlay of Rs.1.60 lakhs is proposed

for the above purpose for five years.

iv. Wind mills for Water Pumping  - It is proposed to provide  Rs.20,000/-

per windmill  as additional subsidy over and above  the Central Subsidy

for installing 20 Nos. of  Wind Mill for water pumps in the State during

Tenth Plan.  An outlay of Rs. 4  lakhs is proposed this scheme.

v. Wind – Solar Hybrid Systems - In order to  promote the new concept of

wind  – solar hybrid system in the case of private investors, it is

proposed to give an additional subsidy of  Rs.50,000/-  per KW over  716 11.2 Non-Conventional Sources of Energy

and above Central Subsidy  for a total of 40 KW in multiples of 1 KW.

during the Tenth Plan. An outlay of Rs.20 lakhs is proposed for this

scheme.

During Tenth Plan, a total outlay of Rs.35.60 lakhs has been proposed

for Wind Power generation  programmes.

B.  Bio-Energy

i.  Bio-Gas Plant - The Bio-Gas development is being undertaken as a

State Level Programme for community and institutional set up.   It is proposed

to instal 10 numbers of Bio-Gas Plants in Sugar Mills using pressmud with 60

cum capacity. A provision of Rs. 11.50 lakhs  has been proposed as State

Subsidy Component during the Tenth Five Year Plan.

ii. Community Bio-gas Plants (CBP)- TEDA has installed 9 Community

Biogas Plants under State Funding.  It is proposed to maintain these plants  at

an annual cost of Rs.12,000/- per year per plant through Non-Governmental

organizations such as Rotary Club, etc.  An outlay of Rs.5.40 lakhs is

proposed for this scheme.   A total outlay of Rs.16.90 lakhs has been

proposed for the Bio-Gas Plants Programme during Tenth Plan.

iii.  Gasifier - It is proposed to  install 50 Nos. of Gasifiers of 100 Kwe

capacity with a State subsidy at 20% of the cost of Rs. 5  lakhs each.  The

Gasifiers are proposed to be installed in Educational Institutions for cooling

applications.  An outlay of Rs. 50 lakhs is proposed for this scheme during the

Tenth Plan.

iv. Energy Park  - The establishment of Energy Parks in various

institutions will be continued under MNES programme.  It is proposed to

establish a State Level Energy Park at Chennai with financial assistance upto

Rs. 1 Crore from Ministry of Non-conventional Energy Sources and the State

Government's share of  Rs. 60 lakhs towards the cost of civil works in

buildings etc., and a minimum of 3 to 5 acres of land in TEDA’s name, having

easy access to Public. Further seminar halls etc. can also be built in these in

order to generate sufficient revenue component for maintaining the energy

park as self supporting.  The objectives of the Energy Park will  include

generating awareness amongst common people about renewable and nonconventional resources of energy, energy conservation practices and to

attract entrepreneurs towards renewable energy based projects.

A total outlay of Rs.126.90 lakhs has been proposed for the Bio-Energy

Programmes during the Tenth Plan period.

C.1.  Solar Energy

(i)  Solar Photovoltaic Lantern - During Tenth Plan, it is proposed to

distribute 1,000 lanterns per year to the poor people like fishermen.  The

above SPV Lanterns will emit light covering 360 degrees and will be capable

of lighting for 3 hours for a CFL 7 watts lantern or 4 hours for a CFL 5 watts

lantern with an average daily solar  radiation of 5 kwhr / sq.mt. of  horizontal

surface. The cost of a lantern is Rs.4,000/-  which includes the cost of

module, electronics and lamp accessories.   It is proposed to provide 100%

State subsidy for this scheme.  The total cost  will be Rs. 40 lakhs / annum.  11.2 Non-Conventional Sources of Energy 717

The number of beneficiaries during Tenth Plan will be 5,000.    An outlay of

Rs. 200 lakhs is proposed for this scheme during the Tenth Plan.

(ii) Solar Photovoltaic Pump - Against a back log of 4,70,000

applications for service connection for agricultural pump-sets with TNEB, only

40,000 could be covered per year.  The Board is also suffering commercial

losses due to free supply of power to them apart from the line losses of

around 20%.  Hence SPV pump is the most reliable and cost effective

alternative option.   It has no battery and easy to maintain and the life of solar

module which converts solar energy directly into electricity is more than 10

years.  A 900 watt pump cost Rs.2.24 lakhs, of which Rs.50,000/- will be

borne by the beneficiary, Rs.99,000/- (at Rs.110/- per watt) by MNES and

Rs.75,000/- is proposed as State subsidy.  It is proposed to instal 70 SPV

pumps every year with a total outlay of Rs.53.20 lakh/ year including

Rs.1000/- per pump as service charges to TEDA.  The total outlay for Tenth

Plan is Rs.266 lakhs.

(iii)  SPV Street Lights - Like other SPV programmes,  it is proposed

to install 100 SPV Street Lights per year  in selected areas in consultation with

District Collectors in the industrial premises, parks, gardens, educational,

charitable institutions etc.  The cost of each Street Light is Rs.30,000.  The

subsidy component is Rs.11,000/- per light and Rs.500 will be charged by

TEDA as service charge.  The balance cost will be met by the beneficiary

organization.  During the Tenth Plan, it is proposed to instal 500 SPV Street

Lights. An outlay of Rs.57.50 lakhs is proposed for this scheme.

(iv)  SPV Home Lighting - It is proposed to instal SPV Home Lights in

selected buildings in consultation with District Collectors in industrial

premises, tourist places etc.  During Tenth Plan, it is proposed to instal 200

Home lights per year with a  subsidy of Rs.5500/- per system and  Rs.200/-

each as service charges to TEDA.  The balance cost will be met by the

beneficiary organization.  This scheme provides easy and quick method for

electrification of households in remote and difficult villages and hamlets in the

State.  During Tenth Plan it is proposed to instal 1000 numbers of  SPV

Home Lighting  Systems.  An outlay of Rs.57 lakhs is proposed for this

programme during Tenth Plan.

(v) Repayment of IREDA Loan - During 1993-94, the Government of

Tamilnadu sanctioned installation of 50 SPV pumps, for which Government

has undertaken to repay the IREDA loan of Rs.40 lakhs at the rate of

Rs.80,000/- per pump and interest at 2.5% thereon  over a period of ten

years. All the 50 SPV pumps have been installed.   Up to Ninth Plan, eight

instalments have been paid and the remaining 2 instalments will have to be

paid during  Tenth Plan, which works out to  Rs.8.28 lakhs.    An evaluation

study of the above 100 SPV pumps conducted by Anna University during

1998-99 showed that 73% of the pumps are working satisfactorily and the rest

were having minor problems  due to non-maintenance of the pumps.  A

provision of Rs.8.28 lakhs is proposed for this scheme as loan component.

(vi)  Electrification for remote habitations (matching grant by  State

Govt.) - TNEB has identified 122 Nos. of remote hamlets spread over 11

districts in Tamilnadu, which could not be electrified by conventional grid

supply. Electrification of the above hamlets is proposed during the Tenth Five 718 11.2 Non-Conventional Sources of Energy

year Plan. The cost of electrification of each hamlet by providing Solar lighting

devices is estimated at Rs.10 lakhs. MNES was requested to provide 90%

cost with a matching grant proposed at 10% by State Government.  However,

MNES  has indicated that their funding may only be up to 50% cost. Hence

the State Government will have to meet the balance 50% cost. As a provision

of only Rs.122 lakhs is proposed for the State Government during 10

th

Five

Year Plan, the number of hamlets to be electrified will be correspondingly

restricted.

An total outlay of Rs.710.78  lakhs has been proposed for Solar

Photovoltaic   Energy during  Tenth  Plan.

C.2.  Solar Thermal Energy

i.  Solar Water Heating System (SWHS) -  During the  Tenth Five Year

Plan, it is proposed to instal 700  Domestic SWHS and 35 Nos. industrial

system of capacity 1000 LPD systems per year with a  subsidy of Rs.3000 per

unit and Rs.300 as service charges  to TEDA, at a total cost of Rs.23.10 lakh

every year is proposed.  The number of beneficiaries during Tenth Plan will be

3500 nos.  An outlay of Rs.115.50 lakhs is proposed for the scheme for Tenth

Plan.  Similarly, for 35 nos. of Industrial SWHS,  a State subsidy of Rs.20000

per unit besides service charges of Rs.2000 to TEDA with a total cost  of

Rs.7.70 lakhs per year is proposed .  The target fixed for this scheme is 175

numbers.  An outlay of Rs.38.50 lakhs is proposed for the scheme during

Tenth Plan.

ii. SWHS in Government buildings  - It is proposed to instal   Solar

Water  Heating Systems (SWHS)  of 2000 LPD 80°C at a cost of Rs.4.00

lakhs per unit in Government hospitals / hostels/ institutions  with 100% fund

assistance. The total cost the programme works out to Rs.200 lakhs.  During

Tenth Plan period, 50 numbers   are proposed to be installed.    An outlay of

Rs.200 lakhs is proposed for this scheme.

iii. Solar Stills - Solar stills can produce potable water to cater to the

drinking water and cooking water needs of fishermen families by making use

of the freely available solar energy.  To popularise the use of this device, it is

proposed to instal 12.5 sq. metre area of solar stills each year for  producing

the potable/ distilled water.   The number of units could be decided based on

its area during implementation.  This scheme  will cater to  the needs of

State Transport Corporations, fishermen  colonies   and rural people  for

producing potable drinking water and distilled water from  brackish water.  It is

estimated  that, one sq.metre area of Solar stills can produce  to 4 litres of

potable / distilled water per day.    Tentatively, system cost is fixed at

Rs.2000/- per LPD and 50LPD is proposed to be installed.   The subsidy

component for this scheme works out to Rs. 25000/-  and service charge  to

TEDA  will be Rs.5000/-.  It is proposed to instal 250 LPD during the Tenth

Plan. An outlay of Rs.1.50 lakhs is proposed for this scheme during the Tenth

Plan.

iv. Solar Passive buildings  -The  working and living spaces in idential

and non residential buildings can save millions of Kw hrs. of electrical energy

by keeping buildings cold during summer and warm during winter by

incorporating various available energy saving features and concepts in the11.2 Non-Conventional Sources of Energy 719

design and construction of buildings at the conceptual stage. For creating an

awareness about the concept, MNES is at present offering incentives for

preparation of Detailed Project Report (DPR) to a maximum of Rs.50,000 or

1.5% of the estimated cost for the Architecture design with Solar Passive

Concept with the incentive for construction of Buildings limited to 10% of the

cost of the buildings or maximum of Rs.10 lakhs for each project. Likewise,

the State Government can extend support to promote the solar passive

concept.  Hence, for the introduction of solar passive concept, State

Government needs to provide incentives for the construction of Government,

Semi Government buildings from 2002-2007 onwards.   It is proposed to

provide Rs 25,000/- per DPR to prepare DPRs for 25 projects and Rs. 5 lakhs

per building covering 10 buildings for construction of a Solar Passive

Architecture System with service charges of Rs.1 lakh per year to TEDA

during the Tenth Plan.  An outlay of Rs.61.25 lakhs has been proposed for

this programme during  Tenth Plan.

v. Solar Air Heating Systems (SAHS) - In Tamilnadu  'Solar Air Heating

Systems' are introduced as an alternate to “Open drying” due to the limited

fossil fuel reserves and its ever increasing demand.  The Solar Air Heating

System consists of  an array of solar collectors generally installed over the

roof of buildings which converts ambient air into very hot air with an increase

of 30-40°C temperature.  The above pre-heated air is used in tea drying/ fruit

process/ grain drying industries and this process fetches 25-30% fuel saving.

It is proposed to offer a subsidy for installation of SAHS to a total area of 1000

sq.m at Rs.1000/sq.m annually.  The proposed target during the Tenth Five

Year Plan is 5000 Sq.m.  An outlay of Rs.52.50 lakhs is  proposed for this

scheme for Tenth Plan.

vi. Solar Cooker  -To encourage the use of environment friendly Solar

Cooker, a subsidy of 20% of the cost of the cooker plus a Service charge of

Rs.100 per cooker to TEDA is proposed under this scheme.  The target fixed

during 10

th

Plan is 3000 nos. of Solar Cookers. An outlay of Rs.20.50 lakhs is

proposed for this scheme during Tenth Plan.

A total outlay of Rs.489.75 lakhs has been proposed for Solar Thermal

Energy Programmes  during 10

th

Plan period.

D. Waste to Energy Programme

MNES, Government of India has sanctioned the setting up of a 0.5

MW power project from waste water from Varalakshimi Starch Industries

Limited at Pappireddipatty under UNDP/ GEP assisted project and is

providing financial support up to 50% of the cost of the project of Rs. 359 lakh

for installation and execution through TEDA.  The civil work has been

completed and the project was expected to be commissioned by end of  2002.

MNES/ Government of India have also sanctioned another proposal for

setting up 1.2 MW power project from poultry waste at Sarkar Uduppam

village in Namakkal district by M/s.G.K.Bio Energy Private Ltd., Chennai at a

cost of Rs.1402 lakhs and is providing financial assistance of Rs. 351 lakh,

and IREDA sanctioned a loan of Rs. 641 lakh. 70% of Civil works were

completed and the project is expected to be completed by December 2002.720 11.2 Non-Conventional Sources of Energy

E. Integrated Rural Energy Programme (IREP)

This on-going Programme will be continued in the existing 21 blocks in

18 districts during the Tenth Plan.  The various energy saving devices such as

chullah, cookers, lighting systems etc. are to be distributed to the rural people

at subsidy ranging from 25 to 100%.  The scheme cost has been proposed  at

Rs.5 lakh per block as indicated by MNES. An outlay of Rs.525 lakhs for

scheme cost and Rs.152.90 lakhs for staff cost has been proposed for this

scheme under Tenth Plan.  An additional amount of Rs.250 lakhs has also

been provided under rural energy programme for implementing schemes

under international funding assistance.  The total outlay will be Rs. 927.90

lakhs in the Tenth Plan.

F. Publicity

TEDA has been conducting Seminars, Workshops and Exhibitions

mainly through sponsorships offered by the Ministry of Non-conventional

Energy Sources, Government of India.  It is essential that the publicity is

confined not only to the urban public but it should be taken to the grass root

level in rural areas.

During Tenth Plan, an outlay of Rs.50 lakhs has been proposed for

carrying out the following activities by the TEDA:

(i) Publicity through various media-postal, AIR  and TV.

(ii) Exhibitions, Seminars, Workshops, Business meets, training

programmes etc.

(iii) Publication of books, periodicals manuals etc. on renewable energy

gadgets, success stories.

(iv) Opening authorised Service Centres in each district for  Renewable

energy devices

(v) Supporting Energy conservation measures in various colleges/

schools and conduct of regular interactive meetings

(vi) Efforts to include in the curricula subjects/chapters on Renewable

Energy in Schools and Colleges

(vii) Setting up a Renewable Energy Fair in a reputed Education

Institute,

(viii) Conducting three months' short time certificate programme on

Maintenance and upkeep or Renewable Energy Gadgets, and

(ix) Adopting a model village where all renewable energy gadgets are

put to use.

An outlay of Rs. 50 lakhs has been proposed during the Tenth Plan

period for the above purpose.

G. Evaluation Studies

TEDA does  periodical evaluation of the programmes implemented in

the State during Plan periods.  It is proposed to conduct various evaluation

studies relating to Non-conventional Energy Sources during the Tenth Plan.   11.2 Non-Conventional Sources of Energy 721

An outlay of  Rs. 24 lakhs  has been proposed for this programme

during Tenth Plan.

H. Research & Development

The main objective of  the Research and Development Programme in

TEDA will be to conduct and promote R & D in the fields of renewable energy

sources.   During Tenth Plan it is proposed to start R & D programmes  in

coordination with institutions like Anna University, M.K. University, Regional

Engineering College, Trichy, PSG College of Technology, Coimbatore, and

continue the same with increased activities in the subsequent years.   R & D

will also envisage  generation of funding assistance from Ministry of Non  –

conventional Energy Sources  and departments like National Research

Development Programme and venture to generate earnings in future  through

commercialization to ensure self-support.  In addition to the R & D efforts

proposed,    Department  officials  will be permitted  to participate in

International Seminars / Training Programmes / Study tours in order to

develop their technical knowledge.   An outlay of Rs.100 lakhs has been

proposed in the Tenth Plan for R & D Programmes.

I. Establishment Cost

The establishment  cost  includes salary component, T.A and other

charges for TEDA.   It is proposed to strengthen the organisation with

permanent technical staff to take up consultancy services to generate regular

revenue  to TEDA and thereby lessen the burden of the State Government.

An outlay of   Rs. 500 lakhs  is proposed  towards establishment charges for

TEDA  during  the  Tenth  Plan.

In all, a total outlay of Rs.2965.00 lakhs is proposed for implementing

the programmes pertaining to Non-conventional Energy Sources during the

Tenth Plan period.

Recommendations/ Suggestions

a) A consistent and  long term policy may be formulated and adopted to

promote harnessing of renewable energy sources in the State.

b) A Single Window Agency may be established under TEDA to act as a

forum for speedy clearance of  projects.

c) Tariff may be restructured and power purchase policy revised suitably

to make investment in renewable energy attractive for investors taking

into account not just the economic cost but also the social/

environmental costs associated with conventional power sources.

d) The grant of suitable fiscal and financial incentives may be considered

for investment in renewable sources of energy taking into account the

clean and green nature of the energy source such as exemption from

sales tax or sales tax deferral for investment in renewable energy

projects

e) Merger of NBDP and IREP schemes along with staff may be

undertaken so as to create field outfits to promote renewable energy

projects/ schemes722 11.2 Non-Conventional Sources of Energy

f) Publicity and propaganda may be expected to create better awareness

about advantages of  renewable energy sources.

g) R & D may be encouraged and supported so that the efficiency of

devices could be improved and cost brought down.

h) Energy efficiency and conservation may be given high priority - TEDA

can play an important role if it is  notified as the Designated Agency

under the Energy Conservation Act.

i) Electrification of remote habitations without grid power may be

undertaken using renewable sources.

j) Private investment may be explored in new sources of renewable

energy such as micro hydel projects.

Annexure – I

Yearwise Tenth Plan  (2002-07) -Financial Outlay

(Rs in lakhs)

Scheme 2002-03 2003-04 2004-05 2005-06 2006-07 Outlay

I.WIND ENERGY

Micro Study  of

remaining Windy

Station and

further Wind

Monitoring

Stations  through

C-WET

2.00  2.00 2.00 2.00 2.00 10.00

2. Maintenance

of 3 permanent

Wind Monitoring

Stations

0.32 0.32 0.32 0.32 0.32 1.60

3.  Installation of

Wind Mills for

water pumping

1.00 1.00 1.00 1.00 4.00

4. Wind Solar

Hybrid System

5.00 5.00 5.00 5.00 20.00

Total –I Wind

Energy

2.32 8.32 8.32 8.32 8.32 35.60

II.  SOLAR

ENERGY

1. SPV Lanterns

40.00

.

40.00 40.00 40.00 40.00 200.00

2. SPV Pumps 53.20 53.20 53.20 53.20 53.20 266.00

3. SPV Street

Lights

11.50 11.50 11.50 11.50 11.50 57.50

4. Repayment of

IREDA loan

4.19 4.09 -- -- -- 8.28

5. SPV Home

Light Systems

11.40 11.40 11.40 11.40 11.40 57.00

6. Electrification

of    hamlets

-- 61.00 61.00 -- - 122.00

Total  –II  –Solar

Energy

120.29 181.19 177.10 116.10 116.10 710.7811.2 Non-Conventional Sources of Energy 723

Scheme 2002-03 2003-04 2004-05 2005-06 2006-07 Outlay

III.  SOLAR

THERMAL

ENERGY

1. Domestic Solar

Water Heating

Systems

23.10 23.10 23.10 23.10 23.10 115.50

2. Industrial Solar

Water Heating

Systems

7.70 7.70 7.70 7.70 7.70 38.50

3.  Installation of

Solar Water

Heating Systems

in Government

Hospitals/

Institutions

40.00 40.00 40.00 40.00 40.00 200.00

4.  Installation of

solar stills   0.30 0.30 0.30 0.30 0.30 1.50

5.  Solar Passive

Architecture

System

12.25 12.25 12.25 12.25 12.25 61.25

6.Solar Air

Heating Systems

10.50 10.50 10.50 10.50 10.50 52.50

7.  Solar Cooker * 4.10 4.10 4.10 4.10 4.10 20.50

III.Total  -  Solar

Thermal Energy

97.95 97.95 97.95 97.95 97.95 489.75

IV.  BIO  –

ENERGY -

1. Biogas

Industries Sugar

Mills

2.30 2.30 2.30 2.30 2.30 11.50

Maintenance of 9

CBPs

1.08 1.08 1.08 1.08 1.08 5.40

2.Gassifier

Cooking

application for

Education

Institutions

10.00 10.00 10.00 10.00 10.00 50.00

3.  State Level

Energy Park

0.00 0.00 60.00 0.00 0.00 60.00

Total – IV- BioEnergy

13.38 13.38 73.38 13.38 13.38 126.90724 11.2 Non-Conventional Sources of Energy

Scheme 2002-03 2003-04 2004-05 2005-06 2006-07 Outlay

V.  INTEGRATED

RURAL ENERGY

PROGRAMME

1.  Continuance

of IREP in 21

blocks – for

devices            -

for staff

105.00

30.58

105.00

30.58

105.00

30.58

105.00

30.58

105.00

30.58

525.00

152.90 *

Sub - Total  135.58 135.58 135.58 135.58 135.58 677.90

2.  Matching

grant (10%) for

IREP under

International

funding

Assistance

50.00 50.00 50.00 50.00 50.00 250.00

Total – V –

IREP

185.38 185.38 185.38 185.38 185.38 927.90

VI.  Publicity 10.00 10.00 10.00 10.00 10.00 50.00

VII. Evaluation

studies

0.00 6.00 6.00 6.00 6.00 24.00

VIII.Research

and Devp

20.00 20.00 20.00 20.00 20.00 100.00

IX.   Establishment cost

70.00 80.00 100.00 120.00 130.00 500.00

GRAND TOTAL

499.52 607.42 683.33 582.33 592.33

2964.93 or

2965.00

*  Funded by Government of India

Annexure - II

Year Wise   Tenth Plan (2002-2007)   Physical Targets

Schemes

Unit 2002-

03

200

3-04

2004-

05

2005-

06

2006

-07

TOTAL

X Plan

Target

A. Wind Energy MW - - - - - 300

B. Cogeneration,

Bagasse and

Biomass based

power genertion

MW - - - - - 300

I.WIND ENERGY

1. Micro Study  of

remaining Windy

Station and

further Wind

Monitoring

Stations  through

C-WET

Nos. 1  1  1  1  1  5 11.2 Non-Conventional Sources of Energy 725

Schemes

Unit 2002-

03

2003

-04

2004-

05

2005-

06

2006

-07

TOTAL

X Plan

Target

2.  Installation of Wind

Mills for water pumping

Nos. - 5  5  5 5  20

3. Wind Solar Hybrid

System

KW. - 10  10  10  10  40

II. SOLAR ENERGY

1. SPV Lanterns

Nos. 1000  1000  1000  1000  1000  5000

2. SPV Pumps Nos. 70  70  70  70  70  350

3. SPV Street Lights  Nos. 100  100  100  100  100  500

4. SPV Home

Light Systems

Nos. 200  200  200  200  200  1000

5. Electrification

of     hamlets

Nos. - 61  61  - -    122

III. SOLAR  THERMAL

1. Domestic Solar

Water Heating Systems

Nos. 700  700 700 700  700  3500

2. Industrial Solar

Water Heating Systems

Nos. 35  35  35  35     35  175

3.  Installation of Solar

Water Heating Systems

in Government

Hospitals/Institutions

Nos. 10    10  10  10  10 50

4.  Installation of solar

stills

LPD 50  50  50  50  50  250

5.  Solar Passive

Architecture System

DPR

Buildin

gs

5

2

5

2

5

2

5

2

5

2

25

10

6.Solar Air Heating

Systems

Sq.M. 1000  1000 1000  1000  1000  5000

7.  Solar Cooker   * Nos. 600 600  600  600 . 600  3000

IV  BIOENERGY:

1. Bio gas

Industries Sugar Mills

Nos. 2  2  2  2  2  10

2. GASIFIER

Cooking application for

Education Institutions

Nos. 10  10  10  10  10  50

3. State Level Energy

Park

Nos.   - - 1  - - 1

V. IREP

Continuance of IREP in

21 blocks

No.of

Blocks

21  - - - - 21 726 11.2 Non-Conventional Sources of Energy

Annexure – III

Specific  Monitorable  Targets

The following specific targets are envisaged for the tenth plan for

propagating non-conventional renewal sources of energy.

Sl.

No

Scheme Units Targets

1. Micro Study of Remaining Wind Stations  Nos. 24

2. Wind Mill Water Pumps Nos. 20

3. Wind Solar Hybrid System KW 40

4. SPV Lantern Nos. 5000

5. SPV Pumps Nos. 350

6. SPV Street Lights Nos. 500

7. SPV Home Lights Nos. 1000

8. Electrification of Remote Areas Habitates

in

Numbers

.

122

9. Domestic Solar Water Heaters Nos. 3500

10. Industrial Solar Water Heaters Nos. 175

11. Solar Water Heating System (Govt. Buildings) Nos. 50

12. Solar Stills LPD 25

13. Solar Passive Architecture System DRPs

Buildings

25

10

14. Solar Air Heating Systems Sq.M 5000

15. Solar Cooker (Box Type)

(Dish Type)

Nos.

Nos.

500

100

16. Bio Gas Plant Nos. 10

17. Gasifiers (100 KW) Nos. 50

18. State Level Energy Park Nos. 1

19. IREP  – Continuance of IREP in 21 Blocks:  Rural

Energy Programme with International Funding

Assistance.

Nos. 21

1. Introduction to energy

The energy of a body is its capacity to do work.  It is measured the total

amount of work that the body can do.

Energy is the primary and most universal measure of all kinds work by

human beings and nature.  Every thing what happens the world is the

expression of flow of energy in one of its forms.

1.2. Different forms of energy

The different forms of energy are:

1. Mechanical energy (kinetic and potential)

2. Thermal (or) Heat energy

3. Chemical energy

4. Electrical energy

5. Nuclear energy

6. Electromagnetic energy

7. Gravitational energy

The S.I unit of energy is Joule or KJ or Watt.h.

Primary Energy Sources

Primary energy sources can be defined as sources which provide a net

supply of energy Coal, Oil, Uranium etc., are examples of this type. The

energy required to obtain these fuels is much use than what they can produce

by combustion or nuclear reaction.  The supply of primary fuels is limited. It

becomes very essential to use these fuels sparingly.

Examples

Coal, natural gas, oil and nuclear energy.

Secondary Energy sources

Secondary fuels produce no net energy.  Though it may by necessary

for the economy, these may not yield net energy. Non-Conventional Energy Sources

2

Secondary sources are like sun, wind, water (tides), etc.  Solar energy

can by used through plants, solar cells, solar heaters and solar collectors.

1.3. Energy sources and their availability

Introduction

Today every country draws its energy needs from a variety of sources.

We can broadly categorize these sources as commercial and noncommercial.  The commercial sources include the fossil fuels (coal, oil and

natural gas), hydro-electric power and nuclear power, while the noncommercial sources include wood, animal waste and agricultural wastes.  In

an Industrialized country like U.S.A. most of the energy requirements are met

from commercial sources, while in an Industrially less developed country like

India, the use of commercial and non-commercial sources are about equal.

1.4. Conventional and non-conventional sources energy

1.4.1. Conventional energy sources

Conventional  energy sources are:

(a) Fossil fuel energy

(b) Hydraulic energy

(c) Nuclear energy

(a) Fossil fuel energy

Coal, petroleum, and natural gas are called Fossil fuel as these are

formed by the decomposition of the remains of dead plants and animals

buried under the earth for a long time.  These are non-renewable sources of

energy, which, if exhausted, can not be replenished in a short time.  Their

reserves are limited and are considered very precious.  These should be used

with care and caution to let them last long.  These are also contributing to the

global environmental pollution.

(i) Coal

Since the advent of industrialization coal has been most common

source of energy.  In the last three decades, the world switched over from Non-Conventional Energy Sources

3

coal to oil as a Major source of energy because it is simpler and cleaner to

obtain useful energy from oil.

Coal is a complex mixture of compounds of carbon, hydrogen and

oxygen.  Small amounts of nitrogen and Sulphur compounds are also present

in coal.  It is mainly available in Bihar, West Bengal, Orissa and Madhya

Pradesh.   The big coal mines in our country are at Jharia and Bokaro in Bihar

and at Raniganj in West Bengal.  It is considered as the backbone of the

energy sector for its use in industry, transportation and electric power

generation.

Depending upon the carbon contents of coal, it is classified as follows:

Type of coal Carbon content (%)

1. Peat 60

2. Lignite (soft coal) 70

3. Bituminous (house hold coal) 80

4. Anthracite (hard coal) 90

It is clear that peat is the most inferior quality of coal.  Whereas

anthracite is the most superior quality of coal.  Bituminous is most commonly

used in households and industry.

On strong heating, coal breaks up to produce coal gas, ammonia,

coaltar and coke.  Coke is 98% carbon, obtained after lossing all its volatile

constituents during destructive distillation of coal.  It can be used as smoke

free fuel.

(ii). Petroleum

It is a dark coloured, viscous and foul smelling crude oil.  The

petroleum means rock oil.  It is normally found under the crust of earth

trapped in rocks.  The crude oil is a complex mixture of several solid liquid

gaseous hydrocarbons mixed with water, salt and earth particles.  It is a

natural product obtained from oil wells.  Non-Conventional Energy Sources

4

Some of the crude oil producing locations in our country are:

(i) Ankleshwar and  Kalol in Gujarat

(ii) Rudrasagar and Lakwa in Assam; and

(iii) Bombay high (off-shore area)

The oil wells of Bombay high are producing about 22 million tons of

crude petroleum oil per year, which is little less than half of the total

requirement of the country.  The efforts are also being made to search oil well

in off-shore deltas to Godavari, Kaveri, and Rajasthan.

The crude petroleum is refined by the process of fractional distillation to

obtain more useful petroleum products.  The crude petroleum is heated to a

temperature of about 400

o

C in a furnace and vapors thus formed are passed

into a tall fractioning column from near its bottom.  As the mixture of hot

vapours rises in the column, it starts getting cooled gradually.

The products obtained from crude petroleum as follows:

(i) Petroleum gas (below 40

o

C) used as LPG.

(ii) Petrol (40

o

C to 170

o

C) for light vehicles.

(iii) Kerosene (170 to 250

o

C) for household and industrial use.

(iv) Diesel oil (250 to 350

o

C) for heavy vehicles.

(v) Residual oil ; (a) Lubrication oils (b) paraffin wax and (c) asphalt

(vi) Fuel oil (350 to 400

o

C) for boilers and furnaces.

(iii) Natural gas

It consists about 95% Methane and rest ethane and propane.  It occurs

deep under the crust of the earth either alone or a long with oil above the

petroleum deposits.  It is a product of petroleum mining.

The gas is available in Tripura, Jaisalmer, off-shore areas of Bombay

High and in the Krishna – Godavari delta.  It is used as a domestic and

industrial fuel.  The natural gas is now also available as CNG (Compressed

Natural Gas) a substitution of petrol in automobiles.Non-Conventional Energy Sources

5

(b) Hydraulic energy (or) Water power

Water power is developed by allowing water to fall under the force of

gravity.  It is used almost exclusively for electric power generation, in fact, the

generation of water power on a large scale became possible around the

beginning of the twentieth century only with the development of electrical

power plants or Hydro electric plants were usually of small capacities usually

less than 100 KW.

Potential energy of water is converted into Mechanical energy by using

prime moves known as hydraulic turbines.  Water power is quite cheap where

water is available in abundance.  Although capital cost of hydro electric power

plants is higher as compared to other types of power plants but their operating

Costs are quite low, as no fuel is required in this case.  The development rate

of hydropower is still low, due to the following problems:

1. In developing a project, it will take about 6-10 years time for planning,

investigation and construction.

2. High capital investment is needed, and some parts of the investment have

to be designed from foreign sources.

3. There are growing problems on relocation of villages, involved,

compensation for damage, selecting the suitable resettlement area and

environmental impact.

Because of long transmission line to the villages with low load factor,

the electric power will be available to the people in rural areas may not be

economical.  This leads to the development of Mini or Micro hydroelectric

projects to supply the electric power to remote areas.  The Importance of

Micro hydroelectric projects have been observed in some parts of the country

with availability of river flow through out the year.

In order to reduce the cost of development several Measures have

been considered as follows:

(a) Development of low cost turbines and generators.

(b) Participation of villages in the development and operation of the project.

(c) Using the appropriate technology and tolerable substandard requirement

and project civil work component at the beginning stage. Non-Conventional Energy Sources

6

(c) Nuclear energy

According to modern theories of atomic structure, matter consists of

minute particles known as atoms.  Heavier unstable atoms such as U

235

,

Th

239

, liberate large amount of heat energy.  The energy released by the

complete fission of one Kg of Uranium (U

235

), is equal to the heat energy

obtained by burning 4500 tonnes of coal (or) 220 tons of oil.  The heat

produced by nuclear fission of the atoms of fissionable material is utilized in

special heat exchangers for the production of steam which is then used to

drive turbogenerators as in the conventional power plants.

However there are some limitations in the use of nuclear energy

namely high capital cost of nuclear power plants, limited availability of raw

materials, difficulties associated with disposal of radio active waste and

shortage of well trained personnel to handle the nuclear power plants.

The Uranium reserves in the world at present are small.  These

reserves are recoverable but are expensive. The presently working power

plants are:

1. Tarapur atomic power station in Maharashtra

2. Ranapratap sagar  atomic power station near Tota, Rajasthan

3. Kalpakkam atomic power station near Madras, Tamilnadu.

4. Narora atomic power station in U.P.

About 3% of the energy produced in India is obtained from nuclear

power plants.

1.4.2. Non-Conventional Energy Sources

The sources of energy which are being produced continuously in

nature and are in exhaustible are called renewable sources of energy (or)

non-conventional energy.

Some of these sources are:

(a) Wind energy

(b) Tidal energy

(c) Solar energy Non-Conventional Energy Sources

7

(a) Wind energy

Winds are caused because of two factors.

1. The absorption of solar energy on the earth’s surface and in the

atmosphere.

2. The rotation of the earth about its axis and its motion around the Sun.

A wind mill converts the kinetic energy of moving air into Mechanical

energy that can be either used directly to run the Machine or to run the

generator to produce electricity.

(b) Tidal energy

Tides are generated primarily by the gravitational attraction between

the earth and the Moon.  They arise twice a day in Mid-Ocean.  The tidal

range is only a Meter.

Basically in a tidal power station water at high tide is first trapped in a

artificial basin and then allowed to escape at low tide.  The escaping water is

used to drive water turbines, which in turn drive electrical generators.

(c) Solar energy

Brief history of solar energy (or) Importance of solar energy:

Energy from the sun is called solar energy.  The Sun’s energy comes

from nuclear fusion reaction that take place deep in the Sun.  Hydrogen

nucleus fuse into helium nucleus.  The energy from these reactions flow out

from the sun and escape into space.