KPCL Electrical Question Paper for Assistant and Junior Engineer 
1. Mho relay is usually employed for the protection of 
a) Short lines only 
b) Medium lines only 
c) Long lines only 
d) Any line 
Ans: Long lines only 
2. A modern power semiconductor device IGBT is combines the characteristics of 
a) BiT and MOSFET 
b) ECT and GTO 
c) SCR and MOSFET 
d) SCR and BiT 
Ans: BIT and MOSFET 
3. For a single-phase a.c. to d.c. controlled rectifier to operate in regenerative mode, which of the following conditions should be 
satisfied? 
a) Half —controlled bridge, a 900, source of e.m.f. in load 
c) Full-controlled bridge, a > 90°, source of e.m.f. in load 
d) Full-controlled bridge, a 900, source of e.m.f. in load 
4. Which is the most suitable power device for high frequency ( more 100 kHz) switching application? 
a) Power MOSFET 
b) BIT 
c) SCR 
d) UJT 
Ans: Power MOSFET 
5. In a thyristor Latching current is than Holding current 
a) Equal 
b) Less 
c) Greater 
d) None 
Ans: Greater 
6. The transfer function of a system is 10/(1+s) when operating as a unity feedback system, the steady state error to a step input

will be 
a) 0 
b) 1/11 
c) 10 
d) Infinity 
Ans: 1/11 
7. Which one of the following statements for a dc machine which is provided with inter pole winding (1W) as well as compensating 
winding (CW) is correct 
a) Both 1W and CW are connected in series with Armature winding 
b) Both 1W and CW are connected in parallel with Armature winding 
c) 1W connected in series but CW is connected in parallel with Armature winding 
d) CW connected in series but 1W is connected in parallel with Armature winding 
Ans: Both 1W and CW are connected in series with Armature winding 
8. A 0-10 mA PMMC ammeter reads 5mA in a circuit; its bottom control spring snaps out suddenly the meter will now read.                                                          
a) 5mA 
b) lOmA 
c) 2.SmA 
d) 0 
Ans: 0 
9. A dc cumulatively compounded motor delivers rated load torque at rated speed. If series field is short circuited, then the 
armature current and speed will 
a) Both increases 
b) Both decreases 
c) Increases and decreases 
d) Decreases and increases 
Ans: Both increases 
10. Moving coil in dynamometer wattmeter connected 
a) In series with fixed coil 
b) Across supply 
c) In series with load 
d) Across load 
Ans: Across supply 
11. In an induction machine, if the air gap increased 
a) Speed will be reduced 
b) Efficiency will be improved 
c) Power factor will be lowered 
d) Breakdown torque will be reduced 
Ans: Power factor will be lowered 
12. A CR0 screen has ten divisions on the horizontal scale. If a voltage signal 5 sin 314t+45 deg)is examined with a line base 
settings of 5 msec/div, the number of cycles of signal displayed on the screen will be 
a) 0.5 cycles 
b) 2.5 cycles 
c) 5 cycles 
d) 10 cycles 
Ans: 2.5 cycles 
13. A 3-phase 50HZ SCIM takes a power input of 30 KW at its full load speed of 1440 rpm. Total stator losses are 1 KW. The slip 
and rotor ohmic losses at full load are 
a) 0.02, 600 W 
b) 0.04, 580 W 
c) 0.04, 1160 W 
d) 0.04, 1200 W 
Ans: 0.04, 1160 W 
14. Thermocouple is used to measure 
a) AC 
b) DC 
c) Both 
d) None 
Ans: Both 
15. The two watt meters measurement the ratio of two meter readings is —(1-sqrt3):( 1+sqrt3) then the power factor is 
a) 1 
b) 0.866 
c) 0.707 
d) 0 
Ans: 0.707 
16. Hays bridge is used to measure____________ and Schering bridge is used to measure_____________ 
a) Inductance, Inductance 
b) Inductance, Capacitance 
c) Capacitance, Inductance 
d) Resistance, Capacitance 
Ans: Inductance, Capacitance 
17. When sine wave is given as input to Schmitt trigger then its generates

a) Sine wave 
b) Saw tooth wave 
c) Triangle wave 
d) Square wave 
Ans: Square wave 
18. In Gauss Seidel method the following factors are influenced for operation 
a) Acceleration factor 
b) Selection of slack buss 
c) Both 
d) None 
Ans: Selection of slack buss* 
19. 
i. (X’÷Y’) A. Low-pass filter function 
H. (X’Y’) B. Sum 
Hi. (XY) C. NAND 
D. Carry 
E.NOR 
a) i—C, H—B, Hi—D 
b) i—C, H—B, Hi—B 
c) i—C, H—B, Hi—D 
d) i—C, H—B, Hi—A 
Ans: i-C, H-B, Hi-D 
20. The phase lead compensation is used to 
a) Increase rise time and decrease overshoot 
b) Decrease both rise time and overshoot 
c) Increase both rise time and overshoot 
d) Decrease rise time and increase overshoot 
Ans: Decrease rise time and increase overshoot 
21. Voltage feed back amplifier is a 
a) Shunt-Shunt 
b) Shunt-Series 
c) Series-Shunt 
d) Series- Series 
Ans: Shunt-Shunt 
22. In microprocessor the next instruction to be executed is stored in 
a) Program Counter 
b) Stack Pointer 
c) Memory Pointer 
d) Accumulator 
Ans: Program Counter 
23. The following element retains it energy after source is disconnected 
a) Resister 
b) Inductor 
c) Capacitor 
d) Thermistor 
Ans: 
24. In series RLC circuit at resonant 
a) Voltage is in phase with current 
b) Current is maximum 
c) Inductive reactance = Capacitive reactance 
d) All of the above 
Ans: All of the above 
2S.For RC low pass filter R=100 K ohms, C= S micro farads then lower cutt of frequency is 
a) 1 K HZ 
b) 0 HZ 
c) 381.3 HZ 
d) Infinity 
Ans: 
26. V=lOOSin (l000t+46 deg), I=2Sin (l000t+80 deg) what are the elements in the circuit 
a) R=30 ohm, L=30 mH 
b) R=30 ohm, C=33.3 micro farads 
c) R=40 ohm, L=30 mH 
d) R=40 ohm, L=33.3 micro farads 
Ans: R=40 ohm, L=33.3 micro farads 
27. L=10 mH, I=lOOSin5Ot 0 
a) 
b) 
c) 
d) 
Ans: 
28. In dielectric measurement, the dielectric loss is proportional to

a) F 
b) V 
c) I 
d) Vsqr 
Ans: Vsqr 
29. G(s) = (1-s)/s(s+2) then closed ioop transfer function is 
a) Unstable 
b) Stable 
c) Marginally stable 
d) All 
Ans: Stable 
30. G(s) = (1+0.Ss)/(1+s) find which type of net work it is 
a) Lead net work 
b) Lag net work 
c) Lag — Lead net work 
d) Lead — Lag net work 
Ans: Lag net work 
31. Temperature of electrode in Arc furnace is 
a) 1000 deg 
b) 1500 deg 
c) 1500 deg to 3500 deg 
d) 4500 deg 
Ans: 
32. In bode plot the point which separates the lower and upper frequencies is called 
a) Critical point 
b) Cut-off point 
c) 
d) 
Ans: 
33. Nyquest stability is used to determine 
a) Absolute Stability 
b) Relative Stability 
c) Both 
d) None 
Ans: Both* 
34. When 220V dc shunt alternator generating the voltage at rated value. If direction of rotation is reversed, then alternator will 
a) Build up its voltage with same polarity 
b) Build up its voltage with opposite polarity 
c) No build up of voltage 
d) None 
Ans: No build up of voltage 
35. Over lap Angle depends on__________ 
a) Load inductance 
b) Loa capacitance 
c) Source inductance 
d) Source capacitance 
Ans: Source inductance 
36. Area under speed time curve gives 
a) Time 
b) Speed 
c) Distance 
d) None 
Ans: Distance 
37. The regulation of Short Transmission lines depends up on 
a) Distance of line 
b) Frequency 
c) Power factor 
d) All 
Ans: Power factor 
38. Which of the following plant is having lowest load factor? 
a) Diesel Plant 
b) Pumped storage Plant 
c) Thermal Plant 
d) Nuclear Plant 
Ans: Diesel Plan 
39. For SR latch whent the out put is undesirable 
a) 0, 0 
b) 1, 0 
c) 0, 1 
d) 1, 1 
Ans: 1, 1

40. For interrupting capacitive currents which Circuit Breaker (CB) is used 
a) SF6 CB 
b) Oil CB 
c) Vacuum CB 
d) Air blast CB 
Ans: Vacuum CB 
41. For V/F control, when frequency is increased in transformer 
a) Core loss component current increases, Magnetizing component current decreases 
b) Core loss component current increases, Magnetizing component current increases 
c) Core loss component current decreases, Magnetizing component current decreases 
d) Core loss component current decreases, Magnetizing component current increases 
Ans: Core loss component current decreases, Magnetizing component current decreases 
42. In ceiling fan the angle between auxiliary winding a main winding is 
a) 0 deg 
b) 90 deg 
c) 180 deg 
d) 360 deg 
Ans: 90 deg 
43. In a shaded pole motor, shaded rings are used to 
a) Field flux production 
b) 
c) 
d) 
Ans: Field flux production 
44. Practical method of improving string efficiency 
a) Increasing crass arms length 
b) Using different size of insulators 
c) Using different insulator materials 
d) Using of guard rings 
Ans: Increasing crass arms length* 
45. In which type of fault all 3-phase components are equal 
a) L-G 
b) L-L 
c) L-L-G 
d) 3-Phase fault 
Ans: 3-Phase fault 
46. 11/220 KV 100 MVA transformers, the primary base voltage rating is 10 KV then secondary base KV is 
a) 10 KV 
b) 220 KV 
c) 220/sqrt3 
d) 
Ans: 220/sqrt3* 
47. Water hamming effect is occurs in 
a) Surge tank 
b) Penstock 
c) Turbine 
d) Reservoir 
Ans: Penstock 
48. Transient stability can be improved by 
a) By putting series capacitor 
b) By using dynamic resister 
c) Auto re-closers 
d) All of the above 
Ans: All of the above 
49. If sending end voltage is Vs at no-load in a transmission line then receiving end voltage is if ABCD parameters of line is given 
a) Vs 
b) Vs/A 
c) 0 
d) Infinity 
Ans: Vs/A 
50. Harmonics are eliminated by using 
a) Skewing of rotor 
b) Distribution winding 
c) Short pitch winding 
d) All of the above 
Ans: All of the above 
51. For parallel operation of transformer at no load, then load shared are equal when 
a) Impedance is proportional with respect to own KVA rating 
b) 
c) 
d)

Ans: Impedance is proportional with respect to own KVA rating 
52. Tn Induction motor Slip frequency of rotor current, when rotor speed is Nr. Then rotor producing slip field rotates with respect to 
stator is 
a) Slip frequency 
b) Nr 
c) Ns 
d) None 
Ans: Ns 
53. Good regulation means 
a) Less fluctuations from no-load to full-load 
b) 
c) 
d) 
Ans: Less fluctuation from no-load to full-load 
54. At dead short circuit at terminals of Alternator then the current I is 
a) ZPF Lag 
b) ZPF lead 
c) Unity power factor 
d) 0.8 Power factor lag 
Ans: ZPF Lag 
55. Full scale Ammeter reading is 10 mA, Voltage across Ammeter is 100 mV. Then for 100 A measurements the power loss in the 
circuit is 
a) 1 W 
b) 10 W 
c) 100 W 
d) 1000 W 
Ans: 10 W* 
56. When Alternator excitation increases and machine is operating at lagging power factor then 
a) I increase 
b) I decreases 
c) No effect on current 
d) None 
Ans: I increase 
57. Flue gases coming out from furnace is first going through 
a) Turbine 
b) Economizer 
c) Air pre-heater 
d) Chimney 
Ans: Economizer 
58. Two alternators rated are 200 KW at 4% regulation, 400 KW S% regulation operating in parallel at 50HZ , when supply 600 KW 
the new frequency is 
a) 50 
b) 49 
c) 51.3 
d) 47.7 
Ans: 47.7 
59. Va and Vb are negative sequence component voltages the difference angle between Va and Vb with respect to Va is 
a) 240 
b) 120 
c) 180 
d) 360 
Ans: 120 
60. 4-Quadrent operation of chopper I is positive but V is may be positive are negative operates in which quadrant 
a) 1 and 2 
b) 2 and 3 
c) 1 and 3 
d) 1 and 4 
Ans: 1 and 4 
61. Heat convection, radiation, and conduction all are takes place in 
a) Ice 
b) Boiler 
c) Refrigerator 
d) Flue gases in pipe 
Ans: 
62. When moving iron meter is used to measure d.c current what is the disadvantage 
a) It calibration in rms value 
b) 
c) 
d) 
Ans:

b3. vvnen maximum power transrer to baa is 
a) 20 W 
b) 50 W 
c) 100W 
d) OW 
Ans: OW 
64. Find Zbus Parameters 
a) Z11=Z12=Z21= jO.1 ohm, Z22=jO.3 ohm 
b) 
c) 
d) 
Ans: Z11=Z12=Z21= jO.l ohm, Z22=jO.3 ohm 
65. Find Z11 and Z12 
a) Z11=4 ohm, Z12= 2 ohm 
b) Z11=4 ohm, Z12= 5 ohm 
c) Z11=5 ohm, Z12= 2 ohm 
d) Z11=4 ohm, Z12= 3 ohm 
Ans: 
66. Current in 5 ohm resister is 
a) 10 A 
h) -10 A 
c) 5 A 
d) -5 A 
Ans: -10 A 
67. Find I in the circuit at t=0+ 
a) 2 A 
b) 5 A 
c) 7 A 
d) 10 A 
Ans: 
8. Find voltage across inductor at t=0+ 
a) 0 V 
b) 2.5 V 
c) 5 V 
d) 10 V 
Ans: 5 V 
69. This wave is applied to the PMMC meter, meter reads 
a) 0 V 
b) 3 V 
c) 2 V 
d) 1 V 
Ans: 1 V 
70. Find Vab 
a) Zero 
b) 50 at an angle of 90 deg 
c) 50 at an angle of 90 deg 
d) None 
Ans: 
71. Find E in the circuit 
a) 5 V 
b) 10 V 
c) 20 V 
d) None 
Ans: None 
72. Vab is reference then angle between Vab and lb is 
a) -15 deg 
b) 240 deg 
c) 180 deg 
d) -45 deg 
Ans: -15 deg* 
63. When maximum power transfer to load is 
Click Here to Show Diagram 
a) 20 W 
b) 50 W 
c) 100W 
d) OW 
Ans: OW 
64. Find Zbus Parameters 
Click Here to Show Diagram 
a) Z11=Z12=Z21= jO.1 ohm, Z22=jO.3 ohm 
b)

C) 
d) 
Ans: 711=712=721= jO.1 ohm, 722=j0.3 ohm 
65. Find 711 and 712 
Click Here to Show Diagram 
a) 711=4 ohm, Z12= 2 ohm 
b) 711=4 ohm, Z12= 5 ohm 
c) 711=5 ohm, Z12= 2 ohm 
d) 711=4 ohm, Z12= 3 ohm 
Ans: 
66. Current in 5 ohm resister is 
Click Here to Show Diagram 
a) 10 A 
b) -10 A 
c) 5 A 
d) -5 A 
Ans: -10 A 
67. Find I in the circuit at t=O+ 
Click Here to Show Diagram 
a) 2 A 
b) 5 A 
c) 7 A 
d) 10 A 
Ans: 2 A 
68. Find voltage across inductor at t=0+ 
Click Here to Show Diagram 
a) 0 V 
b) 2.5 V 
c) 5 V 
d) 10 V 
Ans: 5 V 
69. This wave is applied to the PMMC meter, meter reads 
Click Here to Show Diagram 
a) 0 V 
b) 3 V 
c) 2 V 
d) 1 V 
Ans: 1 V 
70. Find Vab 
Click Here to Show Diagram 
a) Zero 
b) 50 at an angle of 90 deg 
c) 50 at an angle of 90 deg 
d) None 
Ans: 
71. Find E in the circuit 
Click Here to Show Diagram 
a) 5 V 
b) 10 V 
c) 20 V 
d) None 
Ans: None 
72. Vab is reference then angle between Vab and Tb is 
Click Here to Show Diagram 
a) -15 deg 
b) 240 deg 
c) 180 deg 
d) -45 deg 
Ans: -15 deg*