Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2009

Subject Title: 	ENGINEERING MECHANICS								Time : 3 hours

Subject Code:	CE204		       						               		 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define coplanar forces.

2.	When is a particle said to be in equilibrium?

3.	What is a couple?

4.	A sketch of a body isolated from its supports is defined as ______________.

5.	Define coefficient of friction.

6.	What is polar moment of inertia?

7.	Define rectilinear motion.

8.	Write the expression for distance traveled by a body along a straight line.

9.	What is central impact?

10.	Differentiate between kinetics and kinematics.

PART – B  (5 x 3 = 15 MARKS)

11.	If two forces F1 = 20 kN and F2 = 15 kN at an angle 70° act on a particle, find the resultant by parallelogram law.

12.	Briefly explain the various types of supports.

13.	Locate the centroid of the T-section whose flange and web is of size 100 mm x 20 mm.

14.	Differentiate the following: (i) Distance and displacement, (ii) Speed and velocity, and (iii) Acceleration and retardation.

15.	Define the following: (i) Energy, (ii) Potential energy, and (iii) kinetic energy.

PART – C  (5 x 15 = 75 MARKS)

16.	Determine the resultant of the three forces acting on a hook as shown in fig. 1.

y

100 N

80 N

60 N

x

30°

30°

45°

Fig. 1.

(OR)

[P.T.O]

17.	a.	State the parallelogram law of forces with a neat sketch.			   			(5)

b.	A sphere weighing 120 N is tied on to a smooth wall by a string as shown in fig. 2. Find the

tension T in the string and reaction R of the wall.			  					(10)

Fig. 2

18.	Determine the support reactions of the beam shown in fig. 3.

100 N

25 N

40 N

2 m

1.5 m

1 m

1.5 m

A

B

45°

30°

Fig.3

(OR)

19.	Four forces of magnitude 20 kN, 10 kN, 25 kN and 15 kN are acting on a square of side 1.5 m as shown in fig. 4. Determine the magnitude and direction of the resultant. Locate the point of application of resultant force with respect to side AD.

A

B

D

C

30°

45°

60°

60°

15 N

25 N

10 N

20 N

Fig. 4

20.	Using the method of joints, determine the force in each member of the truss shown in fig. 5.

8 kN

3 m

3 m

4 kN

3 m

1.5 m

1.5 m

2 m

A

D

B

E

C

Fig. 5

(OR)

21.	Determine the moment of inertia of an I section shown in fig. 6 about its centroidal  axes.

20mm

300 mm

210 mm

20 mm

100 mm

12mm

Fig. 6

22.	A projectile is aimed at a target on the horizontal plane and falls 12 m short when the angle of projection is 15°, while it overshoots by 24 m when the angle is 45°. Find the angle of projection to hit the target.

(OR)

23.	A train is traveling from A to D along the track. Its initial velocity at A is zero. The train takes 5 minutes to cover the distance AB, 2250 m length and 2.5 minutes to cover the distance BC, 3000m length. On reaching the section C, the brakes are applied and the train stops 2250 m beyond at D. Find,

a.	the retardation on CD

b.	the time taken from A to D

c.	Its average speed for the whole distance.

24.	A stone is dropped from the top of the tower 70 m high. At the same time, another stone is thrown upwards from the foot of the tower with a velocity of 30 m/sec.  When and where do the two stones cross each other?

(OR)

25.	Determine the tension in the strings and the velocity of 1500 N block shown in fig. 7,  5 seconds after starting from,

a.	rest

b.	starting with a downward velocity of 3 m/sec.

Assume pulleys as weightless and frictionless.

500 N

1500 N

Fig. 7