Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November/December 2010

Subject Title:	ENGINEERING MECHANICS 							      Time: 3 hours

Subject Code:	CE204		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Forces exist in pair – State the law which confirms this statement.

2.	Draw the free body diagram of a ladder resting on a rough floor and leaning against a rough wall.

3.	Define “couple” in mechanics.

4.	Name two different supports for a beam.

5.	State any two assumptions made in the analysis of a truss.

6.	Write the expression for finding the radius of Gyration of an area about XX axis.

7.	Name one practical example for uniformly accelerated motion.

8.	Write the expressions for converting angular velocity to linear velocity.

9.	When moving a body horizontally from one point to another, what is the work done against gravitational force?

10.	When is a force said to be impulsive?

PART – B (5 x 3 = 15 MARKS)

11.	Name the fundamental laws of mechanisms.

12.	When is a body said to be in impending motion? State the conditions.

13.	State any three assumptions made in the analysis of projectile.

14.	A particle is projected vertically upwards from the ground with an initial velocity ‘u’ m/s. Write the steps to find the time taken to reach the maximum height.

15.	Define

a.	Line of impact	b.	Direct impact	c.	Oblique impact

PART – C (5 x 15 = 75 MARKS)

16.	Particle ‘o’ is acted by the following forces.

a.	20N inclined 30° to north of east.		b.	25N towards north

c.	30N towards northwest				d.	35N inclined 40° to south of west

Find the resultant force acting on the particle.

(OR)

17.	A system of five forces of 4 kN, 5kN, 6kN, 7kN and 8kN act at one of the angular points of a regular hexagon and pass through the other angular points. Find the magnitude and direction of the resultant of the above system of forces.

18.	Find the support reactions of a simply supported beam of span 10m carrying a uniformly distributed load of 6kN/m over a length of 5m from the left support and a point load of 8kN at the midspan.

(OR)

[P.T.O]

19.	A uniform ladder of weight 1000N and of length 4m rests on a horizontal rough ground and leans against a smooth vertical wall. The ladder makes an angle of 60° with horizontal. When a man of weight 750N stands on the ladder at a distance 3m from the top of the ladder, the ladder is at the point of sliding. Determine the coefficient of friction between the ladder and the floor.

20.	A symmetrical trapezoidal lamina of 40mm top edge and 60 mm bottom edge and a height of 60 mm has a central circular hole of 20 mm diameter. Find the centroid of the lamina by systematic calculations.

(OR)

21.	Find the forces in all the members of the truss shown in the figure.

20°

3 m

3 m

2.25 m

10 kN

20 kN

A

B

C

D

22.	A stone ‘P’ is dropped from the top of a tower of height 40m. At the same time, another stone ‘Q’ is projected vertically upwards with a velocity of 16m/s. At what time and at what height will the two stones cross each other? Also find the relative velocity of stone P with respect to stone Q.

(OR)

23.	A car is negotiating a curved road of radius 800m at a speed of 80km/h. The motorist applies brake suddenly and reduces the speed of the car to 40km/h at a constant rate in 10 seconds. Find the tangential and normal components of the application of brake. Also find the magnitude and direction of the acceleration immediately after the application of brake.

24.	A man of mass 60kg jumps in to a swimming pool, vertically downwards from a height of 19.62m. He goes down in to water by 2m and then starts rising up. Calculate the average resistance of the water.

(OR)

25.	Two weights 80N and 20N are connected by a thread and move along a rough horizontal plane under the action of 40N applied to first weight of 80N.The coefficient of friction between the sliding surfaces of the weights and the plane is 0.3.Determine the acceleration of the weights and the tension in the thread.