Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May 2010

Subject Title:	ENGINEERING MECHANICS 							      Time: 3 hours

Subject Code:	CE204		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Show that the vectors 5i + 8j – 4k and 4i + 2j + 9k are at right angles to each other.

2.	List out the different systems of force.

3.	What is a couple?

4.	What are the different types of supports?

5.	Define centre of gravity.

6.	Define principal moment of inertia.

7.	Define Time of flight of a projectile.

8.	Define work.

9.	Classify the types of friction.

10.	State Coulomb’s law of dry friction.

PART – B  (5 x 3 = 15 MARKS)

11.	Two concurrent forces of 12N and 18N are acting at an angle of 60°. Find the resultant force.

4 KN

8 KN

2 m

4 m

2 m

12.	Find the support reactions of a simply supported beam, shown in figure.

13.	Find the Centroid of the shaded area shown in figure.

Y

r =10cm

30cm

X

O

20cm

14.	A car is moving with a velocity of 20m/s. The car is brought to rest by applying brakes in 6 seconds.	Find (i) retardation  (ii) distance travelled by the car after applying the brakes.

15.	A man weighing 600 N gets into a lift. Calculate the force exerted by him on the floor of the lift, when it is

a.	moving upwards with an acceleration of 3 m/s2, and

b.	moving downwards with the same acceleration.

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	The four coplanar forces are acting at a point as shown in figure. Determine the resultant in magnitude and direction.

Y

156N

24

104N

10

3

X

X'

252N

9

228N

Y'

(OR)

17.	An electric light fixture weighing 150 N hangs from a point C, by two strings AC and BC as shown in figure. Determine the forces in the strings AC and BC.

A

B

45

C

60

150N

18.	A force F= 9i + 3j − 6kpasses through a point A, whose position vector is 4i – 2j + 9k. Find the moment of force about a point B, whose position vector is 6i – 3j- 7k.

(OR)

19.	A forceF = 6i + 2j − 3k acts at A of coordinates (1, 2 ,3). Find the moment of this force about B of coordinates (-2, 3, 4).

20.	Determine the co-ordinates of centroid of the shaded area shown in figure.

Y

30 cm

R = 30 cm

X

30 cm

(OR)

21.	Find the moment of inertia of the section shown below in figure.

Y

60 mm

10 mm

10 mm

G

40 mm

X

X

10 mm

80 mm

[P.T.O]

Y

22.	A train is traveling from A to D along the track shown in figure. Its initial velocity at A is zero. The train takes 5 min to cover the distance AB, 2250 m length and 2.5 minutes to cover, the distance BC, 3000m in length, on reaching the section C, the brakes are applied and the train stops 2250m beyond, at D (a) Find the retardation on CD, (b) the time it takes the train to get from A to D, and (c) its average speed for the whole distance.

t1 = 5 min                      t2 = 2.5 min	                   t3 =?

A	   2250 m  	       B	 3000 m 	    		   C	  	2250 m	       D

(OR)

23.	A particle is projected from a point on an inclined plane with a velocity of 30m/s. The angle of plane and the angle of projection to the horizontal are 15° and 45° respectively. If the motion of the particle is up the plane determine

a.	Time of flight 								b.	range of the projectile

c.	angle of projection for maximum range, and 		d.	the maximum range.

24.	A body of weight 60N is projected up a 15° inclined plane with a velocity of 10m/s. The co-efficient of kinetic friction between the block and the plane is 0.2. Find the maximum distance that the body will move up the plane before it comes to rest.

(OR)

25.	 A sphere of mass 1kg moving with a velocity 2m/s impinges directly on a sphere of mass 2kg at rest. If the first sphere comes to rest after the impact, find the velocity of the second sphere and the co-efficient of restitution.