Total No. of Pages:   2

Register Number:                                                       					       6240

Name of the Candidate:

P.G. DIPLOMA EXAMINATION - 2009

(AUTOMOBILE MAINTENANCE)

(paper – I)

110. BASICS OF AUTOMOBILES

Dec.)                                                                    				        	      (Time: 3 Hours

Maximum: 100 Marks

Answer any FIVE questions
Choosing THREE from Section – A and TWO from Section – B.
All questions carry equal marks.

SECTION - A

| 1. | (a) | Draw a line sketch of a four cylinder four stroke petrol engine and indicate its parts. Mention any of its application | (10) |
| --- | --- | --- | --- |
|  | (b) | Name any five components of a petrol engine. State their functions and also suitable materials for them. | (10) |
| 2. | (a) | Draw the neat sketch of a diesel engine pump and explain its principle of operation | (10) |
|  | (b) | With a neat schematic diagram of a forced lubrication system of an I.C. engine explain its principle of operation | (10) |
| 3. | Describe the working principles of the following systems of I.C. engine. - MPFI systems. - Microprocessor assisted ignition systems. |  | (20) |
| 4. | (a) | With a suitable sketch, explain the working principle of a centrifugal clutch. | (10) |
|  | (b) | Draw a line sketch and explain the various functions of a transmission system of an automobile. | (10) |
| 5. | (a) | With a neat sketch, discuss the construction and working principle of air operated shock absorbers. Also enumerate its merits and limitations. | (10) |
|  | (b) | With a line sketch describe the procedures to design an air conditioning system for a passenger car. | (10) |
| 6. | With suitable diagrams, state the functions of the following components of an automobile. Also enumerate their influence on the smooth working of an automobile. (i) Differential; (ii) Propeller shaft. |  | (20) |

SECTION - B

| 7. | With neat sketches, describe the working principle of the following: (i) Rack and Pinion steering; (ii) Hydraulic brakes. |  | (20) |
| --- | --- | --- | --- |
| 8. | (a) | With a suitable diagram explain the working mechanism of air brake. Also state its advantages and disadvantages. | (14) |
|  | (b) | Discuss the characteristics of a good brake fluid. Also name some of the brake fluids. | (6) |
| 9. | (a) | With a neat sketch, explain the working of a drum brake. | (14) |
|  | (b) | Enumerate the advantages and disadvantages of disc brake over drum brake. | (6) |
| 10. | (a) | Describe various components of a steering gear with a neat sketch. | (10) |
|  | (b) | With a sketch explain the concept of Ackermann steering linkage. Give suitable application. | (10) |

%%%%%%