TataElxsi hardware paper

(1).problem on lissojous pattern: in x-y mode if vlg applied to x-axis is 2 sinwt then what is the vtg to be applied to y-axis so that we observe the following pattern on the cro : [angle b/n x-axis and the diagonal line is 45degrees] ans is 2root 2 sin (wt +45)

(2)what is the resolution of an adc : vref/(2 pow n).

(3)some circuit is given with two f/f s ,what is that circuit? Ans may be shift reg

(4)spatial to temporal converter is ? (a) shift reg ( b) counter (c ) adc ans may be adc

(5) darlington is used for _____ ans high current gain

(6)some circuit with nand gates is given ,what is that circuit ans is xor

(7) a wired circuit with some nand gates is given asked the function of that circuit

(8) an opamp circuit is given asked to find the transfer function of it ( I think its an ideal integrator circuit)

(9)a pole zero pattern of an impedance function is given . it has a conjugate pair of zeros and a conjugate pair of poles ,then what elements does the impedance consists of? Ans LC elements

(10)most computers wont use floating point arithmetic .why?
a.costly b. slow c. cant

(11)bit-slice processor mean …… ans used for shift ,masking of bits

(12)ac motor used in servo applications is (a) single phase induction b.2 phase induction c. 3 phase induction

(13)tv resolution depends on a. bandwidth 2. detector c. no. of frames /sec

(14)equalizing pulses are sent in tv during a. horizontal blanking b. vertical blanking ……….

(15)cdma is not suitable for satellite communication . why ? ans may be complexity of operation

(16)given some waveforms and asked that which of the waveforms will have odd harmonics of sine terms

(17)asked for two’s complement of a number

(18)maximum signed no. represented by a 8 bit computer

(19)a chip has a capacity of 256bits,to implement a 1KB capacity how many chips are required ans 32

(20)ecl operates in ____ and ____ regions of the transistor

(21)given a transient circuit ,asked for time constant while charging and discharging

(22)he had given 4 statements about a f/f and asked which of them are correct
1. it can store a bit 2. jk f/f have rat race when storing a 2 bit info 3. master slave is used to store a 2 bit info
4. d f/f used as a latch for multiplexing operations

(23) Venn diagram is given and asked to write expression for the shared portion in it

(24)cb and ce are ____ biased when transistor is in saturation

( 25) condition for nyquist rate .

(26)carry look ahed adder is 1.fast 2. requires more h/w 3. costly

(27) ideal power supply will have a. zero internal resistance b. more o/p resistance c. less o/p resistance

(28) a problem on cro and probe interface i.e design of an attenuator for the cro probe
(formula for it is R1 * c1 = R2 * C2 )