JINDAL STEEL & POWER LTD.

TECHNICAL SECTION

It consisted of following questions:-

1.       The process of joining two metals is— sintering, thumbling, coining?

2.       Ratchet is provided in micrometer screw gauge to-maintain uniform pressure…

3.       Strain energy is -the energy stored up to elastic limit..

4.       Taper angle is measured by-Sine bar, bevel protector?

5.       A centrifugal governor is – Hartnell, Porter, Proell?

6.       Numerical on Gate design?

7.       Equivalent length in saw is measured from- start point to finish point, from first teeth to last teeth?

8.       Anemometer is used for measuring- temperature of air, velocity of air, viscosity of fluid?

9.       In neutral equilibrium position of meta center is- coincides with center of gravity..

10.   If young’s modulus increases what is the change on thermal coefficient of expansion..?

11.   Kaplan turbine- low head, high discharge..

12.   Strain- (L-l)/L..

13.   Economisor is used for-pre heating feed water..

14.   In reciprocating compressor clearance is provided for- cushioning effect and saves the piston from hitting the cylinder head..

15.   Function of governor is- maintain a constant speed in parting load conditions..

16.   Heat Transfer from hot body to cold body takes place without affecting intervening medium is-radiation..

17.   When conduction will not take place-when temp. of both bodies are equal..

18.   Which of these defines pitch circle diameter-circular pitch, module,diametral pitch(all of these)..

19.   Which of these governor is known as dead weight governor- Portor governor..

20.   Property which resist elastic deformation-stiffness..

21.   Continuous beam is a beam with- more than 2 supports..

22.   More than two supports is known as continuous beam..

23.   For self locking of screw-helix angle > friction angle..

24.   Involute profile is preferred in gears as-it transmits maximum power..

25.   The gear which resists axial thrust-herringbone gears..

26.   For large power transmission, square threads- wear out, less efficient, production is costly..

27.   The weakest part in flange coupling is-key..

28.   Cage is provided in ball bearing to –keep balls in position..

29.   When a mass of critically damped style, degree of freedom of system is deflected from its equilibrium position and then it is released then-it will oscillate with increasing time period.

30.   30(+0.05)(-0.03) is- bilateral type of tolerance..

31.   Numerical on hoop’s stress?

32.   When a cylinder is compressed over another then-hoop stress is reduced.