Model Question Paper

DSCO5 –COMPUTER NETWORKS

Time : 3 hrs		                			                                Maximum marks : 75

Instruction:

1.	Question paper is divided into Group A, Group B and Group C

2.	Each Group is 25 Marks

3.	Figure to the right in bracket indicates marks.

4.	Good Handwriting is expected

5.  	Assume suitable data if necessary.

Group A (25 Marks)

Answer any three questions (Question 1 is compulsory)

Q.1. Give comparison of RS 232-C , V .24 and RS-449.					(5)

Q.2. What  are  different  design  issues  for the layer  ?  Discuss advantages of layered              protocol.										(10)

Q.3. What is the difference in throughput  characteristics of pure & slotted ALOHA.	(10)

Q.4. What are local area networks ? Briefly  describe Local  Topologies .			(10)

Q.5. Explain  distance vector  routing , with am example.				(10)

Group B (25 Marks)

Answer any three questions (Questions 6 is compulsory)

Q.6. A Television   channels are 6 MHz  wide. How  many  bits  per second can be sent if 16 level digital  signals are used ?								(5)

Q.7. Measurements  of a slotted  ALOHA  channel  with an infinite  number of user show  that 10% of  slots  are idle:

- What  is Channel Load

- What is the through put

- Is the channel  under loaded  or  over  loaded ?					(10)

Q.8. Ten 9600 bps  lines are to be multiplied . using  TDM.  Ignoring over head bits, what is the total  capacity required for synchronous  TDM. Assuming  that we wish to limit average line utilization of 0.8  and  assuming  that each line is busy 50% of  the time, what is the  capacity require  for statistical  TDM ?									(10)

Q.9. (i)  A noiseless 10 kHz  Channel  is  sampled every 1msec . What is Maximum data  rate?

(ii) How  many bits of Ram buffer does  a time  switch  interchanges  need I  and I/P   line  samples  are 10 bits  and there are 80 input.						(10)

Q.10. Describe FDDI  &  its  Frame  Format.						(10)

Group C (25 Marks)

All Questions are Compulsory.

Q.11 	Fill in the blanks ( each question carry 2 marks)

- A  network  which is used for  showing  data, software  and hardware among  several users  owing microcomputers  in called--------------------.

- A byte  is always equivalent  to ----------------.

- Various computer  devices in an on-line can  be connected  by  use of-------------------

- In networking  terminology  UTP means-----------------.

- The  slowest  transmission speeds are those of ----------------------.

Q.11	Multiple choice Question (Each question carries 2 marks)

1. Waking of WAN generally involves

- Telephone  line

- microwaves

- satellites

- All of the above.

- A network that requires human inter venation  of route signals is called

- Bus  network

- ring network

- star network

- T-switched network]

3. The slowest  transmission speed are   choice  of

- Twisted pair wise

- coaxial

- Fiber optic  cable

- microwaves

- Which company developed  the TCP/IP protocol  for networking.

- IBM

- DEC

- NOVELL

- DARPA

5. A network  which is   used for showing data, software and hardware  among  several  users  microcomputer is called.

- WAN

- MAN

- LAN

- VAN

Q.11 	True or false (each question carry 1 marks).

- Piggybacking is a transmission technique.

- To connect a computer with a device  in the same room. you might be likely to use a coaxial  cable.

- FDDI is a star network

- Modern is use for multiplexing

- Different  computers are connected to or LAU  by a cable  and telephone line.

*******************