Model Question Paper

DSA2-DSC2-DSCO2-DSE2-DSET2-DSM2 - APPLIED MECHANICS

Time : 3 hrs		                			                                Maximum marks : 75

Instruction:

1.	Question paper is divided into Group A, Group B and Group C

2.	Each Group is 25 Marks

3.	Figure to the right in bracket indicates marks.

4.	Good Handwriting is expected

5.  	Assume suitable data if necessary.

Group A (25 Marks)

Answer any three questions (Question 1 is compulsory)

Q.1. What are the various characterise of  force? 					(5)

Q.2. State and prove the parallelogram of the forces ?					(10)

Q.3. Show that the  algebraic sum of the resolved  parts of a number of forces in a given direction is equal to resolved  parts of their resultant in the same direction.			(10)

Q.4. State Triangle law of the forces  and polygon law of the forces.			(10)

Q.5. Explain clearly the procedure for finding out the resultant  force analytically and graphically.

(10)

Group B (25 Marks)

Answer any three questions (Questions 6 is compulsory)

Q.6. Find the magnitude of two forces, such that if they act at night angles, their resultant is 10  N. But if they act at 600  their resultant is    13 N. 					(5)

Q.7. Derive an expression for Brake power calenlated by rope brake dynamometer.	(10)

Q.8. Derive a relation for minimum force (P) which will keep the body in equilibrium, resting an a rough inclined plane, when it is at the point of sliding downwards. 		(10)

Q.9. Find the force required to move a load of 300 N up a rough plane, the force being applied parallel to the plane. The inclination of the plane is such that when the same load is kept on a perfectly smooth plane inclined at the same angle, a force of 60 N applied at an inclination of 300 to the plane, keeps the same load in equilibrium. 				(10)

Assume coefficient of friction between the rough plane and the load is equal to 0.3.

Q.10. Derive an expression for the efficiency of a worm & worm wheel. 			(10)

Group C (25 Marks)

All Questions are Compulsory.

Q.11 	Fill in the blanks ( each question carry 2 marks)

- Mathematically, coefficient of friction  = ________________.

- The force of friction always acts in a direction _______________ to that in which the body is moving.

- 8.	The velocity   ratio of a single purchase crab wrinch can be increased  by _______________ the length of the handle.

- If the number of pulbys in a system is equal to its velocity ratio then it is a set of ______________ system of pulleys.

- First law of motion states   - V = u + _______.

Q.11	Multiple choice Question (Each question carries 2 marks)

1.	In order to determine the effects of a force acting on a body, we must know : -

(a) Its magnitude 	(b) direction of line  along which it acts.

(c) Its nature (whether push or pull (d) point through which it acts on body

(e) All of the above.

2.	If the resultant of two forces P & Q acts at an angle (x) with P then.

(a)	tan   = P Sin Q 			(b)	Tan    = P Cos Q

P + Q Cos Q 					P + Q Cos Q

(c)	tan    = Q Sin Q 			(d)	tan    = Q Cos Q

P + Q Cos Q 					P + Q Cos Q

3.	The centre of gravity of a  Semi Circle lies at a distance of ____________ from its base measured along vertical radius.

(a)	3r		(b)	4r		(c)	4	(d)	3

4			3			3r		4 r

4.	A circular hole of radius (r) is cut out from a circular disc of radius (2r) in such a way that the diagonal of the hole is the radius of the disc. The centre of grvity of the section lies at :

(a)	Centre of the disc			(b)	Centre of the  hole

(c)	Somewhere in the disc		(d)	somewhere in the hole

5.	The magnitude of the force of friction between two bodies one lying above the other, depends upon the roughness of the.

(a)	upper body 			(b)	Lower body

(c)	both the bodies 		(d)	the body having more roughness

Q.11 	True or false (each question carry 1 marks).

- A force is an agent which destroys or tends to destroy motion.

- The vector method, for the resultant force, is also called polygon low of forces.

- According to Lami's thercm the three forces must be equal.

- Lami's theoram cannot be applied in case of concurrent  forces.

- e C.G. of an equilateral triangle with each side (a) is a/2/3 from any of the three sides.

*********************