Model Question Paper

DSA5 – DSC5 – DSM5 – STRENGTH OF MATERIAL

Time : 3 hrs		                			                                Maximum marks : 75

Instruction:

1.	Question paper is divided into Group A, Group B and Group C

2.	Each Group is 25 Marks

3.	Figure to the right in bracket indicates marks.

4.	Good Handwriting is expected

5.  	Assume suitable data if necessary.

Group A (25 Marks)

Answer any three questions (Question 1 is compulsory)

Q.1. Define strain Explain its different types. 						(5)

Q.2. Derive the formula for change in length of bar by Stress due to rotation. 		(10)

Q.3. Define any one method of finding out M I of composite section. 			(10)

Q.4. Define centre of grarrity . Explain any one method for finding  out C.G.		(10)

Q.5. Derive the formula for variation of shear & tress for rectangular beam. 		(10)

Group B (25 Marks)

Answer any three questions (Questions 6 is compulsory)

Q.6. A steel rod 15 m long is at a temperature of 150c. Find free expansion of length when temperature is raised to 650 C. Find temperature stress produced when expansion of rod prevented. X = 12 10-6 per 0 C & E = 200 GN/ML. 					(5)

Q.7. A copper sleev, 20 mm internal 26 mm external diameter surrounds a 18 mm steel bolt. One end of steev being in contact with the shoulder of bolt. On putting a rigid washer on the other end of the tube a nut is screwed on the bolt until the compressive stress in the sleev is 75 MN/m2. Find  tensile load on steel bolt. take E steel = 200 GN/m2, Ecu = 90 GN/m2. (10)

Q.8. A steel bar 4 cm*4 cm in section,3 m long is subjected to an axial pull of128 KN. Taking E = 200 GN /m2 calculate  change in length of bar. Calculate also the amount of energy stored in bar during extension. 								(10)

Q.9. An unknown weight falls through a height of 10 mm on a collar rigidly attached to a lower end of a vertical bar 5 m long and 600 mm in section. If maximum extension of rod is 2 mm what is stress & magnitude of weight. 							(10)

Q.10. Three beams have the same length, the same allowable stress and  same B.M. The cross section of the beams are a square, a rectangle with depth twice the width and a circle. Determine the ratio of weight of circular beam & rectangular beam w.r.t. the square beam. (10)

Group C (25 Marks)

All Questions are Compulsory.

Q.11 	Fill in the blanks ( each question carry 2 marks)

- Proof resilience is the mechanical property of materials which indicates their capacity to bear .......................... .

- In case of a solid shaft the strain energy in torsion per unit volume is ................

- MI of the rectangle about X X - axis  is ...........................

- MI of circle about YY - axis is .......................... .

- Every body has only .......................... C.G.

Q.11	Multiple choice Question (Each question carries 2 marks)

- The relation between E (Modulus of elasticity) & C (Modulus of rigidity)

  - E = C [ 1 + 1/M ]  (b) E = 2 c [1+1/M]  (c) E = C [1+2/M]

(d) E = 2 C [1 + 2/M]

- The relation between E, K, & C is

(a) E = 2kc 		(b) E = 3k+c  	(c) E = 6kc 	(d) E = 3kc

3K+c 	              6kc   		k+3c	            3k+c

- The poison's ratio for Aluminum is :

(a) 0.13 	(b) 0.33 	(c) 0.43 	(d) 0.43

- The temperature stress developed in a bar depends upon.

  - Co-efficient of linear expansion

  - Change in temperature

  - Young's modulus

  - All above.

- M I of Semi Circle about X X - axis is

(a) 0.22r3 	(b) 0.11 r4	(c) 0.14 r4	(d) 0.2 r4

Q.11 	True or false (each question carry 1 marks).

- The point of contraflexlure is also called as point of inlexion.

- A contineous beam is a beam which has less than two supports.

- A point load is one which spread over a length of beam.

- The most common way of keeping the beam of uniform strength is keeping the width uniform and varying the depth.

- Section modulus of a rectangular section is bd2.

********************