# Introduction to TCP/IP

Summary: TCP and IP were developed by a Department of Defense (DOD) research project to connect a number different networks designed by different vendors into a network of networks (the "Internet"). It was initially successful because it delivered a few basic services that everyone needs (file transfer, electronic mail, remote logon) across a very large number of client and server systems. Several computers in a small department can use TCP/IP (along with other protocols) on a single LAN. The IP component provides routing from the department to the enterprise network, then to regional networks, and finally to the global Internet. On the battlefield a communications network will sustain damage, so the DOD designed TCP/IP to be robust and automatically recover from any node or phone line failure. This design allows the construction of very large networks with less central management. However, because of the automatic recovery, network problems can go undiagnosed and uncorrected for long periods of time.

As with all other communications protocol, TCP/IP is composed of layers:

- IP - is responsible for moving packet of data from node to node. IP forwards each packet based on a four byte destination address (the IP number). The Internet authorities assign ranges of numbers to different organizations. The organizations assign groups of their numbers to departments. IP operates on gateway machines that move data from department to organization to region and then around the world.

- TCP - is responsible for verifying the correct delivery of data from client to server. Data can be lost in the intermediate network. TCP adds support to detect errors or lost data and to trigger retransmission until the data is correctly and completely received.

- Sockets - is a name given to the package of subroutines that provide access to TCP/IP on most systems.

## Network of Lowest Bidders

The Army puts out a bid on a computer and DEC wins the bid. The Air Force puts out a bid and IBM wins. The Navy bid is won by Unisys. Then the President decides to invade Grenada and the armed forces discover that their computers cannot talk to each other. The DOD must build a "network" out of systems each of which, by law, was delivered by the lowest bidder on a single contract.

The Internet Protocol was developed to create a Network of Networks (the "Internet"). Individual machines are first connected to a LAN (Ethernet or Token Ring). TCP/IP shares the LAN with other uses (a Novell file server, Windows for Workgroups peer systems). One device provides the TCP/IP connection between the LAN and the rest of the world.

To insure that all types of systems from all vendors can communicate, TCP/IP is absolutely standardized on the LAN. However, larger networks based on long distances and phone lines are more volatile. In the US, many large corporations would wish to reuse large internal networks based on IBM's SNA. In Europe, the national phone companies traditionally standardize on X.25. However, the sudden explosion of high speed microprocessors, fiber optics, and digital phone systems has created a burst of new options: ISDN, frame relay, FDDI, Asynchronous Transfer Mode (ATM). New technologies arise and become obsolete within a few years. With cable TV and phone companies competing to build the National Information Superhighway, no single standard can govern citywide, nationwide, or worldwide communications.

The original design of TCP/IP as a Network of Networks fits nicely within the current technological uncertainty. TCP/IP data can be sent across a LAN, or it can be carried within an internal corporate SNA network, or it can piggyback on the cable TV service. Furthermore, machines connected to any of these networks can communicate to any other network through gateways supplied by the network vendor.

## Addresses

Each technology has its own convention for transmitting messages between two machines within the same network. On a LAN, messages are sent between machines by supplying the six byte unique identifier (the "MAC" address). In an SNA network, every machine has Logical Units with their own network address. DECNET, Appletalk, and Novell IPX all have a scheme for assigning numbers to each local network and to each workstation attached to the network.

On top of these local or vendor specific network addresses, TCP/IP assigns a unique number to every workstation in the world. This "IP number" is a four byte value that, by convention, is expressed by converting each byte into a decimal number (0 to 255) and separating the bytes with a period. For example, the PC Lube and Tune server is 130.132.59.234.

An organization begins by sending electronic mail to Hostmaster@INTERNIC.NET requesting assignment of a network number. It is still possible for almost anyone to get assignment of a number for a small "Class C" network in which the first three bytes identify the network and the last byte identifies the individual computer. The author followed this procedure and was assigned the numbers 192.35.91.* for a network of computers at his house. Larger organizations can get a "Class B" network where the first two bytes identify the network and the last two bytes identify each of up to 64 thousand individual workstations. Yale's Class B network is 130.132, so all computers with IP address 130.132.*.* are connected through Yale.

The organization then connects to the Internet through one of a dozen regional or specialized network suppliers. The network vendor is given the subscriber network number and adds it to the routing configuration in its own machines and those of the other major network suppliers.

There is no mathematical formula that translates the numbers 192.35.91 or 130.132 into "Yale University" or "New Haven, CT." The machines that manage large regional networks or the central Internet routers managed by the National Science Foundation can only locate these networks by looking each network number up in a table. There are potentially thousands of Class B networks, and millions of Class C networks, but computer memory costs are low, so the tables are reasonable. Customers that connect to the Internet, even customers as large as IBM, do not need to maintain any information on other networks. They send all external data to the regional carrier to which they subscribe, and the regional carrier maintains the tables and does the appropriate routing.

New Haven is in a border state, split 50-50 between the Yankees and the Red Sox. In this spirit, Yale recently switched its connection from the Middle Atlantic regional network to the New England carrier. When the switch occurred, tables in the other regional areas and in the national spine had to be updated, so that traffic for 130.132 was routed through Boston instead of New Jersey. The large network carriers handle the paperwork and can perform such a switch given sufficient notice. During a conversion period, the university was connected to both networks so that messages could arrive through either path.

## Subnets

Although the individual subscribers do not need to tabulate network numbers or provide explicit routing, it is convenient for most Class B networks to be internally managed as a much smaller and simpler version of the larger network organizations. It is common to subdivide the two bytes available for internal assignment into a one byte department number and a one byte workstation ID.

The enterprise network is built using commercially available TCP/IP router boxes. Each router has small tables with 255 entries to translate the one byte department number into selection of a destination Ethernet connected to one of the routers. Messages to the PC Lube and Tune server (130.132.59.234) are sent through the national and New England regional networks based on the 130.132 part of the number. Arriving at Yale, the 59 department ID selects an Ethernet connector in the C& IS building. The 234 selects a particular workstation on that LAN. The Yale network must be updated as new Ethernets and departments are added, but it is not effected by changes outside the university or the movement of machines within the department.

## A Uncertain Path

Every time a message arrives at an IP router, it makes an individual decision about where to send it next. There is concept of a session with a preselected path for all traffic. Consider a company with facilities in New York, Los Angeles, Chicago and Atlanta. It could build a network from four phone lines forming a loop (NY to Chicago to LA to Atlanta to NY). A message arriving at the NY router could go to LA via either Chicago or Atlanta. The reply could come back the other way.

How does the router make a decision between routes? There is no correct answer. Traffic could be routed by the "clockwise" algorithm (go NY to Atlanta, LA to Chicago). The routers could alternate, sending one message to Atlanta and the next to Chicago. More sophisticated routing measures traffic patterns and sends data through the least busy link.

If one phone line in this network breaks down, traffic can still reach its destination through a roundabout path. After losing the NY to Chicago line, data can be sent NY to Atlanta to LA to Chicago. This provides continued service though with degraded performance. This kind of recovery is the primary design feature of IP. The loss of the line is immediately detected by the routers in NY and Chicago, but somehow this information must be sent to the other nodes. Otherwise, LA could continue to send NY messages through Chicago, where they arrive at a "dead end." Each network adopts some Router Protocol which periodically updates the routing tables throughout the network with information about changes in route status.

If the size of the network grows, then the complexity of the routing updates will increase as will the cost of transmitting them. Building a single network that covers the entire US would be unreasonably complicated. Fortunately, the Internet is designed as a Network of Networks. This means that loops and redundancy are built into each regional carrier. The regional network handles its own problems and reroutes messages internally. Its Router Protocol updates the tables in its own routers, but no routing updates need to propagate from a regional carrier to the NSF spine or to the other regions (unless, of course, a subscriber switches permanently from one region to another).

## Undiagnosed Problems

IBM designs its SNA networks to be centrally managed. If any error occurs, it is reported to the network authorities. By design, any error is a problem that should be corrected or repaired. IP networks, however, were designed to be robust. In battlefield conditions, the loss of a node or line is a normal circumstance. Casualties can be sorted out later on, but the network must stay up. So IP networks are robust. They automatically (and silently) reconfigure themselves when something goes wrong. If there is enough redundancy built into the system, then communication is maintained.

In 1975 when SNA was designed, such redundancy would be prohibitively expensive, or it might have been argued that only the Defense Department could afford it. Today, however, simple routers cost no more than a PC. However, the TCP/IP design that, "Errors are normal and can be largely ignored," produces problems of its own.

Data traffic is frequently organized around "hubs," much like airline traffic. One could imagine an IP router in Atlanta routing messages for smaller cities throughout the Southeast. The problem is that data arrives without a reservation. Airline companies experience the problem around major events, like the Super Bowl. Just before the game, everyone wants to fly into the city. After the game, everyone wants to fly out. Imbalance occurs on the network when something new gets advertised. Adam Curry announced the server at "mtv.com" and his regional carrier was swamped with traffic the next day. The problem is that messages come in from the entire world over high speed lines, but they go out to mtv.com over what was then a slow speed phone line.

Occasionally a snow storm cancels flights and airports fill up with stranded passengers. Many go off to hotels in town. When data arrives at a congested router, there is no place to send the overflow. Excess packets are simply discarded. It becomes the responsibility of the sender to retry the data a few seconds later and to persist until it finally gets through. This recovery is provided by the TCP component of the Internet protocol.

TCP was designed to recover from node or line failures where the network propagates routing table changes to all router nodes. Since the update takes some time, TCP is slow to initiate recovery. The TCP algorithms are not tuned to optimally handle packet loss due to traffic congestion. Instead, the traditional Internet response to traffic problems has been to increase the speed of lines and equipment in order to say ahead of growth in demand.

TCP treats the data as a stream of bytes. It logically assigns a sequence number to each byte. The TCP packet has a header that says, in effect, "This packet starts with byte 379642 and contains 200 bytes of data." The receiver can detect missing or incorrectly sequenced packets. TCP acknowledges data that has been received and retransmits data that has been lost. The TCP design means that error recovery is done end-to-end between the Client and Server machine. There is no formal standard for tracking problems in the middle of the network, though each network has adopted some ad hoc tools.

## Need to Know

There are three levels of TCP/IP knowledge. Those who administer a regional or national network must design a system of long distance phone lines, dedicated routing devices, and very large configuration files. They must know the IP numbers and physical locations of thousands of subscriber networks. They must also have a formal network monitor strategy to detect problems and respond quickly.

Each large company or university that subscribes to the Internet must have an intermediate level of network organization and expertise. A half dozen routers might be configured to connect several dozen departmental LANs in several buildings. All traffic outside the organization would typically be routed to a single connection to a regional network provider.

However, the end user can install TCP/IP on a personal computer without any knowledge of either the corporate or regional network. Three pieces of information are required:

- The IP address assigned to this personal computer

- The part of the IP address (the subnet mask) that distinguishes other machines on the same LAN (messages can be sent to them directly) from machines in other departments or elsewhere in the world (which are sent to a router machine)

- The IP address of the router machine that connects this LAN to the rest of the world.

The 7 Layers of the OSI Model

The OSI, or Open System Interconnection, model defines a networking framework for implementing protocols in seven layers. Control is passed from one layer to the next, starting at the application layer in one station, proceeding to the bottom layer, over the channel to the next station and back up the hierarchy.

| Application (Layer 7) | This layer supports application and end-user processes. Communication partners are identified, quality of service is identified, user authentication and privacy are considered, and any constraints on data syntax are identified. Everything at this layer is application-specific. This layer provides application services for file transfers, e-mail, and other network software services. Telnet and FTP are applications that exist entirely in the application level. Tiered application architectures are part of this layer. |
| --- | --- |
| Presentation (Layer 6) | This layer provides independence from differences in data representation (e.g., encryption) by translating from application to network format, and vice versa. The presentation layer works to transform data into the form that the application layer can accept. This layer formats and encrypts data to be sent across a network, providing freedom from compatibility problems. It is sometimes called the syntax layer. |
| Session (Layer 5) | This layer establishes, manages and terminates connections between applications. The session layer sets up, coordinates, and terminates conversations, exchanges, and dialogues between the applications at each end. It deals with session and connection coordination, token management, synchronization and traffic management. |
| Transport (Layer 4) | This layer provides transparent transfer of data between end systems, or hosts, and is responsible for end-to-end error recovery and flow control. It ensures complete data transfer. |
| Network (Layer 3) | This layer provides switching and routing technologies, creating logical paths, known as virtual circuits, for transmitting data from node to node. Routing and forwarding are functions of this layer, as well as addressing, internetworking, error handling, congestion control and packet sequencing. |
| Data Link (Layer 2) | At this layer, data packets are encoded and decoded into bits. It furnishes transmission protocol knowledge and management and handles errors in the physical layer, flow control and frame synchronization. The data link layer is divided into two sublayers: The Media Access Control (MAC) layer and the Logical Link Control (LLC) layer. The MAC sublayer controls how a computer on the network gains access to the data and permission to transmit it. The LLC layer controls frame synchronization, flow control and error checking. |
| Physical (Layer 1) | This layer conveys the bit stream - electrical impulse, light or radio signal -- through the network at the electrical and mechanical level. It provides the hardware means of sending and receiving data on a carrier, including defining cables, cards and physical aspects. Fast Ethernet, RS232, and ATM are protocols with physical layer components. |

# Computer

# Networks

- What are the two types of transmission technology available?

(i) Broadcast 	and 	(ii) point-to-point

- What is subnet?

A generic term for section of a large networks usually separated by a bridge or router.

- Difference between the communication and transmission.

Transmission is a physical movement of information and concern issues like bit polarity, synchronisation, clock etc.

Communication means the meaning full exchange of information between two communication media.

- What are the possible ways of data exchange?

(i) Simplex	(ii) Half-duplex	(iii) Full-duplex.

- What is SAP?

Series of interface points that allow other computers to communicate with the other layers of network protocol stack.

- What do you meant by "triple X" in Networks?

The function of PAD (Packet Assembler Disassembler) is described in a document known as X.3. The standard protocol has been defined between the terminal and the PAD, called X.28; another standard protocol exists between hte PAD and the network, called X.29. Together, these three recommendations are often called "triple X"

- What is frame relay, in which layer it comes?

Frame relay is a packet switching technology. It will operate in the data link layer.

- What is terminal emulation, in which layer it comes?

Telnet is also called as terminal emulation. It belongs to application layer.

- What is Beaconing?

The process that allows a network to self-repair networks problems. The stations on the network notify the other stations on the ring when they are not receiving the transmissions. Beaconing is used in Token ring and FDDI networks.

- What is redirector?

Redirector is software that intercepts file or prints I/O requests and translates them into network requests. This comes under presentation layer.

- What is NETBIOS and NETBEUI?

NETBIOS is a programming interface that allows I/O requests to be sent to and received from a remote computer and it hides the networking hardware from applications.

NETBEUI is NetBIOS extended user interface. A transport protocol designed by microsoft and IBM for the use on small subnets.

- What is RAID?

A method for providing fault tolerance by using multiple hard disk drives.

- What is passive topology?

When the computers on the network simply listen and receive the signal, they are referred to as passive because they don’t amplify the signal in any way. Example for passive topology - linear bus.

- What is Brouter?

Hybrid devices that combine the features of both bridges and routers.

- What is cladding?

A layer of a glass surrounding the center fiber of glass inside a fiber-optic cable.

- What is point-to-point protocol

A communications protocol used to connect computers to remote networking services including Internet service providers.

- How Gateway is different from Routers?

A gateway operates at the upper levels of the OSI model and translates information between two completely different network architectures or data formats

- What is attenuation?

The degeneration of a signal over distance on a network cable is called attenuation.

- What is MAC address?

The address for a device as it is identified at the Media Access Control (MAC) layer in the network architecture. MAC address is usually stored in ROM on the network adapter card and is unique.

- Difference between bit rate and baud rate.

Bit rate is the number of bits transmitted during one second whereas baud rate refers to the number of signal units per second that are required to represent those bits.

baud rate  = bit rate / N

where N is no-of-bits represented by each signal shift.

- What is Bandwidth?

Every line has an upper limit and a lower limit  on the frequency of signals it can carry. This limited range is called the bandwidth.

- What are the types of Transmission media?

Signals are usually transmitted over some transmission media that are broadly classified in to two categories.

- Guided Media:

These are those that provide a conduit from one device to another that include twisted-pair, coaxial cable and fiber-optic cable. A signal traveling along any of these media is directed and is contained by the physical limits of the medium. Twisted-pair and coaxial cable use metallic that accept and transport signals in the form of electrical current. Optical fiber is a glass or plastic cable that accepts and transports signals in the form of light.

b) Unguided Media:

This is the wireless media that transport electromagnetic waves without using a physical conductor. Signals are broadcast either through air. This is done through radio communication, satellite communication and cellular telephony.

- What is Project 802?

It is a project started by IEEE to set standards to enable intercommunication between equipment from a variety of manufacturers. It is a way for specifying functions of the physical layer, the data link layer and to some extent the network layer to allow for interconnectivity of major LAN

protocols.

It consists of the following:

- 802.1 is an internetworking standard for compatibility of different LANs and MANs across protocols.

- 802.2 Logical link control (LLC) is the upper sublayer of the data link layer which is non-architecture-specific, that is remains the same for all IEEE-defined LANs.

- Media access control (MAC) is the lower sublayer of the data link layer that contains some distinct modules each carrying proprietary information specific to the LAN product being used. The modules are Ethernet LAN (802.3), Token ring LAN (802.4), Token bus LAN (802.5).

- 802.6 is distributed queue dual bus (DQDB) designed to be used in MANs.

- What is Protocol Data Unit?

The data unit in the LLC level is called the protocol data unit (PDU). The PDU contains of four fields a destination service access point (DSAP), a source service access point (SSAP), a control field and an information field. DSAP, SSAP are addresses used by the LLC to identify the protocol stacks on the receiving and sending machines that are generating and using the data. The control field specifies whether the PDU frame is a information frame (I - frame) or a supervisory frame (S - frame) or a unnumbered frame (U - frame).

- What are the different type of networking / internetworking devices?

Repeater:

Also called a regenerator, it is an electronic device that operates only at physical layer. It receives the signal in the network before it becomes weak, regenerates the original bit pattern and puts the refreshed copy back in to the link.

Bridges:

These operate both in the physical and data link layers of LANs of same type. They divide a larger network in to smaller segments. They contain logic that allow them to keep the traffic for each segment separate and thus are repeaters that relay a frame only the side of the segment containing the intended recipent and control congestion.

Routers:

They relay packets among multiple interconnected networks (i.e. LANs of different type). They operate in the physical, data link and network layers. They contain software that enable them to determine which of the several possible paths is the best for a particular transmission.

Gateways:

They relay packets among networks that have different protocols (e.g. between a LAN and a WAN). They accept a packet formatted for one protocol and convert it to a packet formatted for another protocol before forwarding it. They operate in all seven layers of the OSI model.

- What is ICMP?

ICMP is Internet Control Message Protocol, a network layer protocol of the TCP/IP suite used by hosts and gateways to send notification of datagram problems back to the sender. It uses the echo test / reply to test whether a destination is reachable and responding. It also handles both control and error messages.

- What are the data units at different layers of the TCP / IP protocol suite?

The data unit created at the application layer is called a message, at the transport layer the data unit created is called either a segment or an user datagram, at the network layer the data unit created is called the datagram, at the data link layer the datagram is encapsulated in to a frame and finally transmitted as signals along the transmission media.

- What is difference between ARP and RARP?

The address resolution protocol (ARP) is used to associate the 32 bit IP address with the 48 bit physical address, used by a host or a router to find the physical address of another host on its network by sending a ARP query packet that includes the IP address of the receiver.

The reverse address resolution protocol (RARP) allows a host to discover its Internet address when it knows only its physical address.

- What is the minimum and maximum length of the header in the TCP segment and IP datagram?

The header should have a minimum length of 20 bytes and can have a maximum length of 60 bytes.

- What is the range of addresses in the classes of internet addresses?

Class A	0.0.0.0		-	127.255.255.255

Class B	128.0.0.0	-	191.255.255.255

Class C	192.0.0.0	-	223.255.255.255

Class D	224.0.0.0	- 	239.255.255.255

Class E	240.0.0.0	-	247.255.255.255

- What is the difference between TFTP and FTP application layer protocols?

The Trivial File Transfer Protocol (TFTP) allows a local host to obtain files from a remote host but does not provide reliability or security. It uses the fundamental packet delivery services offered by UDP.

The File Transfer Protocol (FTP) is the standard mechanism provided by TCP / IP for copying a file from one host to another. It uses the services offer by TCP and so is reliable and secure. It establishes two connections (virtual circuits) between the hosts, one for data transfer and another for control information.

- What are major types of networks and explain?

- Server-based network

- Peer-to-peer network

Peer-to-peer network, computers can act as both servers sharing resources and as clients using the resources.

Server-based networks provide centralized control of network resources and rely on server computers to provide security and network administration

- What are the important topologies for networks?

- BUS topology:

In this each computer is directly connected to primary network cable in a single line.

Advantages:

Inexpensive, easy to install, simple to understand, easy to extend.

- STAR topology:

In this all computers are connected using a central hub.

Advantages:

Can be inexpensive, easy to install and reconfigure and easy to trouble shoot physical problems.

- RING topology:

In this all computers are connected in loop.

Advantages:

All computers have equal access to network media, installation can be simple, and signal does not degrade as much as in other topologies because each computer regenerates it.

- What is mesh network?

A network in which there are multiple network links between computers to provide multiple paths for data to travel.

- What is difference between baseband and broadband transmission?

In a baseband transmission, the entire bandwidth of the cable is consumed by a single signal. In broadband transmission, signals are sent on multiple frequencies, allowing multiple signals to be sent simultaneously.

- Explain 5-4-3 rule?

In a Ethernet network, between any two points on the network ,there can be no more than five network segments or four repeaters, and of those five segments only three of segments can be populated.

- What MAU?

In token Ring , hub is called Multistation Access Unit(MAU).

- What is the difference between routable and non- routable protocols?

Routable protocols can work with a router and can be used to build large networks. Non-Routable protocols are designed to work on small, local networks and cannot be used with a router

- Why should you care about the OSI Reference Model?

It provides a framework for discussing network operations and design.

- What is logical link control?

One of two sublayers of the data link layer of OSI reference model, as defined by the IEEE 802 standard. This sublayer is responsible for maintaining the link between computers when they are sending data across the physical network connection.

- What is virtual channel?

Virtual channel is normally a connection from one source to one destination, although multicast connections are also permitted. The other name for virtual channel is virtual circuit.

- What is virtual path?

Along any transmission path from a given source to a given destination, a group of virtual circuits can be grouped together into what is called path.

- What is packet filter?

Packet filter is a standard router equipped with some extra functionality. The extra functionality allows every incoming or outgoing packet to be inspected. Packets meeting some criterion are forwarded normally. Those that fail the test are dropped.

- What is traffic shaping?

One of the main causes of congestion is that traffic is often busy. If hosts could be made to transmit at a uniform rate, congestion would be less common. Another open loop method to help manage congestion is forcing the packet to be transmitted at a more predictable rate. This is called traffic shaping.

- What is multicast routing?

Sending a message to a group is called multicasting, and its routing algorithm is called multicast routing.

- What is region?

When hierarchical routing is used, the routers are divided into what we will call regions, with each router knowing all the details about how to route packets to destinations within its own region, but knowing nothing about the internal structure of other regions.

- What is silly window syndrome?

It is a problem that can ruin TCP performance. This problem occurs when data are passed to the sending TCP entity in large blocks, but an interactive application on the receiving side reads 1 byte at a time.

- What are Digrams and Trigrams?

The most common two letter combinations are called as digrams. e.g. th, in, er, re and an. The most common three letter combinations are called as trigrams. e.g. the, ing, and, and ion.

- Expand IDEA.

IDEA stands for International Data Encryption Algorithm.

- What is wide-mouth frog?

Wide-mouth frog is the simplest known key distribution center (KDC) authentication protocol.

- What is Mail Gateway?

It is a system that performs a protocol translation between different electronic mail delivery protocols.

- What is IGP (Interior Gateway Protocol)?

It is any routing protocol used within an autonomous system.

- What is EGP (Exterior Gateway Protocol)?

It is the protocol the routers in neighboring autonomous systems use to identify the set of networks that can be reached within or via each autonomous system.

- What is autonomous system?

It is a collection of routers under the control of a single administrative authority and that uses a common Interior Gateway Protocol.

- What is BGP (Border Gateway Protocol)?

It is a protocol used to advertise the set of networks that can be reached with in an autonomous system. BGP enables this information to be shared with the autonomous system. This is newer than EGP (Exterior Gateway Protocol).

- What is Gateway-to-Gateway protocol?

It is a protocol formerly used to exchange routing information between Internet core routers.

- What is NVT (Network Virtual Terminal)?

It is a set of rules defining a very simple virtual terminal interaction. The NVT is used in the start of a Telnet session.

- What is a Multi-homed Host?

It is a host that has a multiple network interfaces and that requires multiple IP addresses is called as a Multi-homed Host.

- What is Kerberos?

It is an authentication service developed at the Massachusetts Institute of Technology. Kerberos uses encryption to prevent intruders from discovering passwords and gaining unauthorized access to files.

- What is OSPF?

It is an Internet routing protocol that scales well, can route traffic along multiple paths, and uses knowledge of an Internet's topology to make accurate routing decisions.

- What is Proxy ARP?

It is using a router to answer ARP requests. This will be done when the originating host believes that a destination is local, when in fact is lies beyond router.

- What is SLIP (Serial Line Interface Protocol)?

It is a very simple protocol used for transmission of IP datagrams across a serial line.

- What is RIP (Routing Information Protocol)?

It is a simple protocol used to exchange information between the routers.

- What is source route?

It is a sequence of IP addresses identifying the route a datagram must follow. A source route may optionally be included in an IP datagram header.